## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mechanical Engg.

Subject: Machine Drawing-II(4035) [Old]

# Time Duration: 4 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Calculate the limits of size for a shaft designated as 35g6 for the following data: Diameter step: 30mm to 40mm Magnitude of tolerance IT6=10i i= 0.45 D + 0.001D microns, where D = D1 x D2 with usual notations (I is in microns, D is in mm) Fundamental deviation = - 9 microns |  |
| b) | Indicate the roughness grade symbols for each of the following Ra values: 25µm, 3.2µm, 0.2µm, 0.1µm |  |
| c) | Explain the meaning of the geometrical tolerances for the components shown in Fig. No. 1. |  |
| d) | Show how the roughness is indicated on the component for the following situations: - Surface to be obtained by any production method - Surface to have a machinery allowance of 2mm - Surface to be obtained by machining with limits of Ra values as 3.2µm and 12.5µm |  |
| e) | Sketch the single line representation of the following pipe fittings: Globe valve, Lateral, Cap. |  |
| f) | Draw the symbol for the following welds: Single bevel butt weld, Double-U butt weld, Square butt weld. |  |
| Q.No.2. | Draw neat proportionate free hand sketches of any two of the following: | 2 x 6 = 12 |
|  | i) Gib and cotter joint - front view with top half in section, side view |  |
|  | ii) Flat belt pulley - front view in section, side view |  |
|  | iii) Union joint - front view in section |  |
| Q.No.3. | Fig. No. 2 shows the details of a footstep bearing. Draw the front view in section and top view. Show at least ten dimensions. | (12) |
|  | OR |  |
| Q.No.3. | Fig. No. 3 shows the assembly of Non-return valve. Draw the following in first angle projection: | (12) |
|  | i) Cover plate – sectional front view, top view |  |
|  | ii) Body – front view in section, top view |  |
|  | iii) Stud – front view Show all dimensions. |  |
|  | -2- | …….2/- |
| Q.No.4. | Fig. No. 4 shows the orthographic views of the sheave of an eccentric. Redraw the views and incorporate the following details: | (12) |
|  | i) Cylindrical surface of  200mm is to be machined and roughness to be indicated by |  |
|  | ii) Face on left side to be machined and roughness to be indicated by  |  |
|  | iii) Tolerance of H7 on  80mm |  |
|  | iv) Roughness on inner surface of hole  80mm to be indicated by  |  |
|  | v) Tolerance of K7 on width of keyway |  |
|  | vi) Axis of hole  80mm to be indicated as datum A |  |
|  | vii) Face on left hand side to be perpendicular to datum A within 0.01mm |  |
|  | viii) Tolerance of e8 on  190mm |  |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Fig. No. 5 shows the orthographic views of a machine component fabricated by welding. Redraw the given views and indicate the appropriate welding symbols. |  |
| b) | Fig. No. 6 shows the single line orthographic views of a piping layout. Draw the single line isometric view of the same. Mention all the dimensions. |  |
| Q.No.6. | Study the working drawing of a gear shown in Fig. No. 7 and answer any eight of the following: | (12) |
|  | i) What is the size of the bore of the gear? |  |
|  | ii) What is the roughness of the inner surface of the bore of the gear? |  |
|  | iii) What is the width of the keyway? |  |
|  | iv) What is the surface roughness of the side faces of the keyway? |  |
|  | v) What is the meaning of 1.6 x 45? |  |
|  | vi) What is the meaning of the geometrical tolerance indicated at (a)? |  |
|  | vii) What is the tolerance of dimension 53.8mm? |  |
|  | viii) What is the meaning of the geometrical tolerance indicated at (b)? |  |
|  | ix) What is the size of the blank required to manufacture the gear? |  |
|  | x) What is the diameter of the hub of the gear? |  |