## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electronics Engg./EC/EI/E/E&EE/COMP/MED

Subject: Digital Electronics-I(4134) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the terms positive and negative logic systems. |  |
| b) | Draw the truth table and symbol of NOR and EXOR logic gates. |  |
| c) | What is a flip flop? Define (i) propagation time (ii) Hold time with reference to a flip flop. |  |
| d) | Differentiate between an asynchronous and synchronous counter. |  |
| e) | (i) How may flip flops are needed to construct a mod 10 counter? (ii) State applications of Counters. |  |
| f) | Define (i) fan-out (ii)Noise margin. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State and prove the De Morgan’s theorems. |  |
| b) | (i) Convert (37)8 into binary (ii) Convert (37)16 into binary (iii) Convert (217)8 into hexadecimal (iv) Convert (101.1)2 into decimal (v) Find 1’s complement of 01100 (vi) Find 2’s complement of 11001 |  |
| c) | Perform the following 8-bit arithmetic using 2’s complement method. - (70)10 + (37)10 - (66)10 – (70)10 |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Minimize the following functions using K-map: - f (A,B,C,D) = Σm (1,3,7,11,15) + d (0,2,5) - f (A,B,C,D) = m (4,5,6,7,8,12)• d (1,2,3,9,11,14) |  |
| b) | Realize the following logic gates using NAND gates only: - NOT (ii) OR (iii) NOR (iv) AND |  |
| c) | Draw the logic diagram of clocked D-flip flop and explain its working with the help of truth table. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the diagram of JK flip flop and explain its working. Draw the truth table and symbol of JK flip flop. |  |
| b) | Draw the logic diagram, logic symbol and explain with the help of truth table the working of RS flip flop using NAND gates. |  |
| c) | What are registers? Explain the four modes of operation with help of block diagram? State some applications of shift registers. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design a mod 6 asynchronous counter. |  |
| b) | Design a mod 8 asynchronous up-down counter. Draw the timing diagram for up counter. |  |
| c) | Draw the circuit diagram of a CMOS NAND gate and explain its operation with the help of truth table. | ……2/- |
|  | -2- |  |
| Q.No.6. | Write short notes on any two: | 2 x 6 = 12 |
| a) | RTL |  |
| b) | DCTL |  |
| c) | HTL |  |

***********