## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mechanical Engg.

Subject: Fluid Power Control(ME627)

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State any three desirable properties of an hydraulic fluid. |  |
| b) | Name any two basic components of a pneumatic system and state their functions. |  |
| c) | Describe the construction of shuttle valve and give its symbols. |  |
| d) | What do you mean by cylinder cushioning? Why is it required? |  |
| e) | State the importance of oil and filter changes for a hydraulic system. |  |
| f) | Give any three probable causes of excessive noise in a hydraulic system. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the construction and working of Radial piston pump. |  |
| b) | Explain the working of spool type 4/2 way Direction control valve giving its sketch. |  |
| c) | Illustrate the use of pilot operated check valve to lock a cylinder giving a neat sketch. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe any two types of mounting arrangements of hydraulic cylinder giving a neat sketch. |  |
| b) | Describe construction and working of valve type hydraulic motor. |  |
| c) | Calculate the force on the piston rod during extension stroke of a double acting cylinder, if the diameter of piston is 80mm and the oil pressure is 150 bar. Also determine the diameter of the piston rod if the force during retraction stroke is half that of extension stroke. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is meant by regenerative circuit? Draw a regenerative circuit and explain its working. |  |
| b) | When do we use an Intensifier? Draw a hydraulic circuit using intensifier and explain its working. |  |
| c) | Draw a pneumatic circuit to illustrate the use of Time Delay Valve and explain its working. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a pneumatic circuit to show OR logic and describe its working. |  |
| b) | Draw a pneumatic circuit to operate double acting cylinder with a 5/2 way valve, with speed control in both directions. |  |
| c) | Describe the following tube joints giving neat sketches: - Flareless join ii) Flanged coupling |  |
|  | -2- | …….2/- |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Advantages and limitations of pneumatics |  |
| b) | Compressed air piping layout |  |
| c) | Quick Exhaust valve |  |
| d) | Construction of hydraulic hoses |  |

***********