## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Automobile Engg.

Subject: Design of Automobile Components(AT703)

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define a Strut & state its application in automobiles. |  |
| b) | Explain briefly what is a positive acting clutch with an example. |  |
| c) | Define factor of safety and list three considerations upon which it depends. |  |
| d) | Define a spring & list its three important functions. |  |
| e) | What are the two main considerations while designing a piston head? |  |
| f) | Calculate the axial tensile load that can be safely applied to M36 bolt, allowing a safe tensile stress of 55 MPa. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is a design process? Give step by step analysis. |  |
| b) | With a neat sketch of stress-strain curve explain: - Yield point ii) Ultimate strength iii) Hooke’s Law iv) Elastic limit |  |
| c) | Explain the selection of material & justifications for automobile components. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steel plate subjected to 2 force of 5KN and fixed to a channel by means of three identical bolts is shown in fig.1. The bolts are made of plain carbon steel 30C8 (Syt = 400N/mm2) and the factor of safety is 3. Determine the diameter of the shank. |  |
| b) | A bracket as shown in fig.2 supports a load of 30KN. Determine the size of the bolts, if the maximum allowable tensile stress in the bolt material is 60Mpa. The distances are L1=80mm, L2=250mm & L=500mm. -2- | …..2/- |
| c) | A wall bracket is attached to a wall by means of four identical bolts, two at A & two at B as shown in fig.3. Assuming that the bracket is held up against the wall and prevented from tipping about point C by all four bolts and using an allowable tensile stress in the bolts as 35N/mm2, determine the size of the bolts on the basis of maximum principal stress theory. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A compression spring made of alloy steel of coil diameter 75mm and spring index 6.0, number of active coils 20 is subjected to a load of 1.25KN. Calculate i) The maximum stress developed in the coil ii) The deflection produced & iii) the spring rate. Take G = 80GPa. G is modulus of rigidity. |  |
| b) | A truck spring has 12 leaves two of which are full length leaves. The spring supports are 1.05m apart and the central band is 85mm wide. The central load is 5.4KN with a permissible stress of 280MPa. Determine the thickness and width of the steel spring leaves. The ratio of the total depth to the width of the spring is 3. Also determine the deflection of the spring. |  |
| c) | Explain: i) Free length of the spring ii) Spring index iii) Wahl’s stress factor with respect to helical compression spring. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A single plate clutch having a single driving plate with contact surfaces of each side is required to transmit 110KW at 1250 r.p.m. The outer diameter of the contact surfaces is 300mm. The coefficient of friction is 0.4. Assuming an uniform pressure of 0.17N/mm2, determine the inner diameter of the friction surfaces. |  |
| b) | A multiple disc clutch has five plates having four pairs of active friction surfaces. If the intensity of pressure is not to exceed 0.127N/mm2, find the power transmitted at 500 r.p.m. The outer and inner radii of friction surfaces are 125mm and 75mm respectively. Assume uniform wear and take coefficient of friction as 0.3. -3- | ….3/- |
| c) | A transmission shaft is supported by two bearings which are 1m apart. Power is supplied to the shaft by means of a flexible coupling which is located to the left of left hand bearing. Power is transmitted from the shaft by means of a belt pulley, 250mm diameter, which is located at a distance of 300mm from the left hand bearing. The mass of the pulley is 20Kg and the ratio of belt tension on tight and slack sides is 2:1. The belt acts vertically downwards as shown in fig.4. The shaft is made FeE300 (Syt=300N/mm2) and the factor of safety is 3. Determine the shaft diameter if it transmits 10KW power at 360 r.p.m. from the coupling to the pulley. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A connecting rod of C.I engine is of circular section. Its length is 550mm. The diameter and stroke of the cylinder are 150mm and 240mm respectively. Determine the diameter of the rod to be used for a F.O.S=3.6 with material having yield point stress of 320N/mm2. |  |
| b) | Design the crown thickness of a cast iron piston for a single acting cylinder four-stroke engine from following data: Cylinder bore = 100mm Stroke =120mm Maximum gas pressure = 5N/mm2 Brake mean effective pressure =0.65N/mm2 Fuel consumption -= 0.227 Kg/KWh Speed =2200 r.p.m. Assume suitable data if required. |  |
| c) | Determine the diameter of hollow push rod made of M.S. of an I.C engine if maximum force exerted by push rod is 1550N. The length of the push rod is 50cm. Take factor of safety = 2.2 and E=210KN/mm2. |  |