## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mining Engg.

Subject: Mine Surveying(MN611) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you understand by limit of errors? |  |
| b) | Write short note on application of remote sensing in surveying. |  |
| c) | Mention any six conventional symbols used in plans as per CMR 1957? |  |
| d) | What are the different graphical methods used in computation of areas? |  |
| e) | How do you predict movement of benches to avoid slope failures? |  |
| f) | Enumerate purpose of correlation survey. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the errors that may occur in any survey work right from the stage of surveying to the point of making plans? What precautions should be taken to guard against these? |  |
| b) | Explain the measurement procedure in EDM. |  |
| c) | Describe the different methods of plotting the plans. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the functions and operation of a total station. What are the benefits? |  |
| b) | Describe a Gyro-theodolite. What are its uses in mines? |  |
| c) | i) Explain the principle of GIS. ii) Write short note on GPS. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | An embankment of width 10m and side slopes 1 in 1½ is required to be made on a ground which is level in a direction traverse to the centre line. The central heights at 30m interval are as follows: 0.85, 1.25, 1.75, 2.50, 1.95, 1.25 and 0.9. Calculate the volume of earth work according to i) the Simpson’s formula ii) the Trapezoidal formula. |  |
| b) | A coal heap on level ground measures 40m x 30m. The side slopes at 1.5 vertical to 1 horizontal and the average height of the heap is 4.5m. Calculate the quantity of coal in the heap assuming 1.10m3 of coal to 1T. |  |
| c) | i) A brick shaft has an internal diameter of 6.5m and the brick work is 50cm thick. Calculate the volume of brick in the shaft which is 80m deep. ii) What are the precautions taken during measurement of coal stock on uneven ground? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain correlation in two shafts by Weisbach triangle method. |  |
| b) | Explain underground traversing by theodolite. -2- | …..2/- |
| c) | Two seams of coal are separated by 42m (vertical) of strata which dip at an angle of 20 to the horizontal. Calculate the length of the cross-measure drift driven to connect them i) if the drift is level ii) if it rises 1 in 10 towards the dip of the seams and iii) if it dips 1 in 2 in the same direction as the seams. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | What are the duties and responsibilities of Mines surveyor? |  |
| b) | Explain a method of enlarging a plan. |  |
| c) | Explain a method of curve setting. |  |
| d) | List out equipments used in stope survey. |  |

***********