## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mechanical Engg./E/FT

Subject: Refrigeration & Airconditioning(5034)/(ME502) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define the term coefficient of performance for a Refrigeration system. |  |
| b) | Name the refrigerants and their operating pressures for the following applications: i) Domestic Refrigerator ii) Ice cream plant iii) Split A/C. |  |
| c) | Represent vapour compression cycle on P-H & T-S diagram for the following case: Vapour is dry and saturated at end of compression. |  |
| d) | Draw a labelled sketch of simple vapour absorption system. |  |
| e) | What factors should be considered while selecting a condenser? |  |
| f) | Define the following terms: i) Dry Bulb temperature ii) Wet bulb temperature iii) Enthalpy of moist air. |  |
| g) | Explain the function of fan in A/C unit. How are fans classified? (only for students of old scheme) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of T-S diagram explain the actual vapour compression cycle. |  |
| b) | A vapour compression refrigerator works between the pressure limits of 60 bar and 25 bar. The working fluid is just dry at the end of compression and there is no under cooling of liquid before the expansion valve. Find C.O.P of cycle for the given data. Pressure in bar; Saturation Temp. in K; Enthalpy (KJ/Kg); Entropy (KJ/KgK). Liquid; Vapour; Liquid; Vapour. 60; 295; 151.96; 293.29; 0.554; 1.0332. 25; 261; 56.32; 322.58; 0.226; 1.2464. |  |
| c) | Draw a neat labelled diagram of Electrolux refrigerator and explain its working. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you understand by Hermetically sealed compressor? Give its advantages. |  |
| b) | Explain flooded type of evaporator with the help of neat sketch. |  |
| c) | Explain with a sketch working of Thermostatic expansion valve. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The atmospheric conditions of air are 25C Dry bulb temperature and specific humidity of 0.01 Kg per Kg of dry air. Find i) partial pressure of vapour ii) R.H iii) D.P.T. |  |
| b) | Explain with a neat sketch a ‘Sling Psychrometer’. |  |
| c) | What is sensible heating and sensible cooling? With help of a Psychrometric chart sketch the processes. | …..2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Give the requirements of good room air distribution system. |  |
| b) | 1 Kg of air at 24C and 70% RH is to be mixed adiabatically in a steady state with 1 Kg of air at 16C and 10% RH. Assuming mixing is to be carried out at constant pressure of 1.0 atmosphere, find the temperature and relative humidity of mixture leaving the device. |  |
| c) | Represent the following processes on this psychrometric chart: i) Cooling and dehumidification ii) Heating and Humidification. |  |
| d) | A fault is detected for a Domestic refrigerator, which is ‘No cooling is taking place’. What do you think are the possible reasons for this fault and how to rectify it? (only for students of old scheme) |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a sketch explain the working of window A/C. |  |
| b) | Explain briefly the following A/C controls i) Temperature control ii) Air movement control. |  |
| c) | With the help of sketch explain working of any one of the duct systems used in A/C field. |  |

***********