## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electrical Engg./E&EE

Subject: D.C. Machines(4109) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define (i) Pole pitch (ii) Coil span (iii) Commutator pitch. |  |
| b) | State three differences between lap & wave winding. |  |
| c) | State the equation for emf induced in DC generator & explain the factors on which it depends. |  |
| d) | Write a short note on current controller of DC welding generator. |  |
| e) | State the necessary conditions to be fulfilled for parallel operation of DC generators. |  |
| f) | Explain the function of a commutator in a DC machine for motoring & generating action. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain working of a simple loop generator. |  |
| b) | Explain the classification of DC generator with reference to connection of field winding and armature. Also write equations for terminal voltage. |  |
| c) | A 4-pole DC shunt generator with lap connected armature supplies a load of 100A at 200V. The armature resistance is 0.1Ω & shunt field resistance is 80Ω. Find (i) Total armature current (ii) Emf generated. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a short note on various losses occurring in a DC generator. |  |
| b) | Write a short note on welding generator. |  |
| c) | With as net diagram explain the procedure to connect an incoming DC generator in parallel with the Bus-bar. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw neat connection diagrams for DC–shunt, DC-series & DC-cumulative compound motors & state their voltage equations. |  |
| b) | State the different methods for speed control of DC shunt motor. Explain any one method with a neat diagram. |  |
| c) | With a neat diagram explain construction & operation of 3-point starter. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain principle of operation of DC motor. |  |
| b) | Compare DC series & DC shunt motors with reference to starting torque, speed variation, operation at no load & applications. |  |
| c) | Explain the speed-torque characteristics of a DC-series motor with a neat diagram. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is armature reaction? Explain its effect on performance of DC machine. -2- | …….2/- |
| b) | Explain principle of commutation & reasons for poor commutation. Also explain any one method to improve commutation. |  |
| c) | Explain the process of voltage build up in DC shunt generator. |  |

***********