## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Civil Engg./CT

Subject: Construction-II(4009) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the functions of a foundation in a structure? |  |
| b) | Draw a neat sketch of an isolated footing and a combined footing for R.C.C. columns. Indicate the steel reinforcements. |  |
| c) | In what situations the pile foundation becomes necessary? What are the basic two classification of piles based on the way of transferring the load to the soil strata? |  |
| d) | What are the cofferdams? Explain in brief and mention the types. |  |
| e) | What are the two major troubles/problems encountered while sinking the caissons? Explain in brief with sketches if necessary. |  |
| f) | Write any 4 advantages of steel piles and some disadvantages. |  |
| g) | When it becomes necessary to go for dewatering of trenches? Explain in brief on the procedure of dewatering. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With neat labelled sketches, explain in brief the methods of timbering the trenches. |  |
| b) | What are the problems for foundations in black cotton soil? With neat sections, explain in brief the methods of construction of foundations in black cotton soil. |  |
| c) | Draw neat plan and section of steel grillage foundation. Label the components. Specify where they are suitable. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Where the raft foundations are suitable? Draw neat proportionate plan and section of a typical raft foundation for about 12 columns showing the reinforcements. |  |
| b) | Enlist the complete classification of piles and types. Explain with sketches the methods of construction of any two types of cast-in-situ piles. |  |
| c) | Make list of any six comparisons (advantages and disadvantages) between cast in situ piles and precast piles. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Differentiate between pneumatic caissons and open caissons. Enlist the precautions to be taken while constructing the pneumatic caissons. |  |
| b) | i) Explain the design concepts of retaining walls showing different loads and their directions. ii) Draw neat sections of various types of retaining walls showing position of main reinforcement. iii) What are keys in retaining wall? |  |
| c) | What is underpinning? When this operation becomes necessary? Explain point wise the precautions to be taken while processing underpinning operation. | …..2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write any 6 points on considerations to be made in planning a building to minimize the occurrence of fire breakout and to minimize the damage in case of fire in a building. |  |
| b) | With neat sketches, explain in brief waterproofing methods for floor and walls in basement construction where there are chances of water seepage. |  |
| c) | Write short notes on making a building sound proof and thermal insulation. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Cableways |  |
| b) | Cranes and various types |  |
| c) | Dredgers and types |  |
| d) | Concrete vibrators |  |
| e) | Timber grillage foundation |  |

***********