## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electronics Engg./EC

Subject: Programmable Logic Controller(EN611)

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define PLC. List any four advantages of PLC. |  |
| b) | What is optoisolator? Explain need of optoisolator in PLC? |  |
| c) | Draw the ladder rungs represented by the following Boolean expressions: i) Y=A⋅B ii) Y=A+B+C⋅D |  |
| d) | Explain the difference between safe and unsafe emergency stop system in PLC. |  |
| e) | Explain the concept of battery backed relays. |  |
| f) | What are cascaded timers? Explain the need of cascading in PLC. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat block diagram, explain the processor section of the PLC in detail. |  |
| b) | With the help of a neat diagram, explain the following with reference to PLC: - AC input unit ii) DC input unit |  |
| c) | Explain sourcing and sinking with reference to input devices connected to a PLC. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) State 3 major features of IEC-1131-3 standard. ii) Draw Ladder rungs representing the following: - A Lamp is to be switched on if there is an input from sensor A or sensor B. - Two switches are normally open and both have to be closed for a motor to operate. |  |
| b) | Explain any 3 data comparison instructions used in PLC, with an example ladder diagram for each. |  |
| c) | Write down any 6 rules to be followed while drawing a ladder diagram. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain how a pulse timer is used in PLC to produce a fixed duration of output with the help of an example of ladder diagram. |  |
| b) | Explain the function of a count-up counter with an example of ladder diagram. |  |
| c) | i) List the different types of timers used in PLC. Which one of them is most commonly used? ii) Draw the ladder diagram to represent the following logical functions: - AND 2) EX-OR 3) NOT |  |
|  | -2- | …..2/- |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the function block of Bit shift left and Bit shift right Instruction and explain the difference between the two. |  |
| b) | Explain the concept of internal relays used in PLC with the help of a Ladder diagram example. |  |
| c) | i) What are subroutines? List any 2 subroutine related instructions in PLC. ii) An output is switched on 10 sec after receiving the input signal and will remain on until the input signal in removed. Draw the ladder diagram for this application. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Commissioning a PLC |  |
| b) | PLC system design |  |
| c) | Emergency stop relays |  |
| d) | Functional Block diagram |  |

***********