## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electronics Engg./EC/EI/MED

Subject: Analog Electronics(4140)/(EN405) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State four configurations of Differential amplifier. Also state the advantages of Differential amplifier. |  |
| b) | Draw symbol & equivalent circuit of opamp. Also state the equation for output voltage. |  |
| c) | Why open loop opamp circuits cannot be used for linear applications? |  |
| d) | Draw & explain opamp subtractor circuit. |  |
| e) | Differentiate between active filters & passive filters. |  |
|  | only for students of revised scheme: |  |
| f) | Draw neat block diagram of IC 555. |  |
| g) | Define following terms: - Line regulation - Load regulation |  |
|  | only for students of old scheme: |  |
| f) | Draw block diagram of PLL as a frequency multiplier. |  |
| g) | Explain the operation of Precision rectifier circuit. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circuit diagram & explain the operation of DIBO differential amplifier. |  |
| b) | With the help of block diagram explain the operation of opamp. |  |
| c) | Define following terms: i) CMRR ii) Bandwidth iii) Slew rate iv) Input offset voltage v)Input offset current vi)Input bias current |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw neat block diagram & circuit diagram of voltage series feedback amplifier. State the equation for voltage gain of this circuit & gain of feedback circuit. |  |
| b) | Draw the circuit diagram of closed loop inverting amplifier. Calculate the values of the resistors used in the circuit if required gain of the amplifier is 10. |  |
| c) | Explain the operation of opamp as summing, scaling & averaging circuit. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the operation of V to I converter having grounded load. |  |
| b) | State the limitations of basic integrator circuit. Explain how these limitations are taken care of in practical integrator circuit. |  |
| c) | With the help of circuit diagram & waveforms explain the operation of Schmitt trigger. |  |
| d) | Explain the operation of phase shift oscillator using neat circuit diagram. (only for students of old scheme) |  |
|  | -2- | …..2/- |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design Ist order High pass Butterworth filter having cutoff frequency of 3KHz & pass band gain of 6. Using frequency scaling technique, redesign the above filter to have cutoff frequency of 2KHz. |  |
| b) | Draw the circuit diagram of first order low pass Butterworth filter. Draw ideal & practical frequency response of the same. State the equation for cutoff frequency. Also explain the operation of above filter using gain magnitude equation. |  |
| c) | With the help of circuit diagram, explain the operation of triangular wave generator. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the features & circuit diagram of IC 79XX series fixed voltage regulator. |  |
| b) | Design IC 723 voltage regulator having following parameters. Vin = 15V, Vo = 3.7V, Io = 45mA, Im = 80mA & Vs = 0.7V. |  |
| c) | Design IC 555 Astable multivibrator generating 10KHz output signal & having duty cycle of 40%. (only for students of revised scheme) |  |
|  | only for students of old scheme: |  |
| c) | Explain transfer characteristics of PLL. |  |
| d) | Explain the concept of amplitude scaling & time scaling in Analog computer. |  |

***********