## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Civil Engg./CT/MNE

Subject: Surveying-I(4001)/(CE301) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the following classification of survey: i) Cadastral survey ii) Mine survey |  |
| b) | State various factors considered while selecting survey stations. |  |
| c) | State advantages and disadvantages of compass survey. |  |
| d) | Define the terms: i) Line of colimation ii) Change point iii) TBM. |  |
| e) | Explain what do you understand by ‘Fly levelling’. |  |
| f) | What are the precautions to be taken in Campass survey? |  |
| g) | Explain the methods of orientation of ‘Plane Table’ by Back sighting. (only for students of revised scheme) |  |
| g) | What do you understand by ‘Zero circle’ in planimeter? (only for students of old scheme) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the aid of neat sketch explain methods of measuring the distance along the sloping ground. |  |
| b) | Describe the method you adopt to range the line when end stations are not intervisible due to rising ground between them. |  |
| c) | With the aid of neat sketch describe the procedures of booking of field notes and various entries made in booking field notes. |  |
|  | -2- | ….2/- |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The following are the observations of cross staff survey of a field ABCDEFGA. Calculate the area of plot in hectares. All dimensions given are in metres. D 700 600 90E C75 470 275 110F B60 160 90 30G A 0 |  |
| b) | Following bearings were observed with a compass traverse. After detecting the stations affected by local attraction, find the correct bearings. Line FB BB AB 72° 0 252° 0 BC 89° 30 269° 30 CD 154° 30 333° 30 DE 78° 30 0 30 EF 196° 0 15° 30 |  |
| c) | Following FB were observed in running a closed compass survey: Line FB PQ 80° 30 QR 170° 30 RS 260° 30 SP 350° 30 Calculate the interior angles of closed traverse. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Following bearings were taken in traversing with compass. Line; FB; BB. AB; S45° 30 E; N45° 30 W. BC; S60° 0 E; N60° 40 W. CD; S5° 30 E; N 3° 20 W. DE; N 4° 30 W; S 6° 0 E. Which are the stations affected by local attractions? Determine the corrected FB and BB of each line. -3- | ….3/- |
| b) | The following consecutive readings were taken with a dumpy level of 4m levelling staff on a continuously sloping ground at 30m interval. 0.750; 1.250; 1.785; 2.620; 2.980; 3.400; 1.100; 1.575; 2.135; 2.970; 0.670; 0.890; 1.470; 2.305. If the first reading was taken on a B.M. with RL = 100.000, enter the readings on a level page and after determining RLs of all points, calculate the gradient of the line joining 2nd point and the last point. |  |
| c) | In running Fly levels from a BM of RL=150.000, the following readings were obtained. B.S; 1.315; 2.035; 1.980; 2.625. F.S; 1.150; 3.450; 2.225. From the last position of instrument five pegs at 25m intervals are to be set out on a uniform falling gradient of 1 in 50. The first peg is to have RL of 148.000. Workout the staff readings required for setting the top of pegs at given gradient. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the sources of errors in levelling work? Explain in detail necessary precautions required to eliminate such errors. |  |
| b) | Describe booking and reduction of levels by i) Rise and fall method ii) Height of collimation method. Compare both the methods. |  |
| c) | The following readings were taken in a levelling work. After taking 2nd, 4th and 8th readings the instrument was shifted. 0.725; 1.360; 2.250; 1.290; 2.780; 3.140; 3.920; 0.270; 1.690; 2.250; 3.230. The first reading was taken on BM whose RL is 125.000. Enter the readings in level page and calculate the RLs of all points by H.I. method. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Find the height Bottom of a beam above the floor level from the following data: RL of floor level 205.000 Staff reading on the floor level + 0.705 Staff reading touching the bottom of beam - 3.730 |  |
| b) | Convert the following i) From QB to WCB ii) From WCB to QB 1) N26° 30W; S 38° 30 E 2) 216° 30 108° 20 -4- | ……4/- |
| c) | Following bearing were observed in clockwise traverse of a compass survey. Line FB AB 45° 0 BC 150° 0 CA 270° 0 Calculate the interior angles. |  |
| d) | Draw sketches to represent the following in a survey map: i) Orchard ii) Road on Embankment iii) Bridge Carrying Railway line above road iv) Gate with compound wall |  |
| e) | Explain the method of “Resection” used in plane Table survey. (only for students of revised scheme) |  |
| e) | Describe the use of Abney level. (only for students of old scheme) |  |

***********