## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Drawing(4234)/(FE302) [Old/Rev]

# Time Duration: 4 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of structural Engineer’s Handbook Sp:6(1) & IS 800

is permissible.

5) All dimensions in the figures are in ‘mm’ unless specified.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you understand by Flange and Girder? |  |
| b) | Explain the term Driving clearance. |  |
| c) | Sketch any one type of foundation bolt. |  |
| d) | Explain what is fabrication drawing. |  |
| e) | State the advantages of Bill of Material. |  |
| f) | Draw free hand sketches of any two types of roof trusses. |  |
| g) | What is the need of pipe bends in pipelines? |  |
| Q.No.2. | Draw the fabrication drawing of gusset plate at ‘A’ and ‘B’ and determine the exact length of member A-B of the portion of truss shown in fig.1. | (12) |
|  | Note: i) All members are ISA 6060 x 6mm ii) Snap head rivets of 14mm are used iii) Gusset plate thickness is 8mm |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Sketch a Double riveted single strap zig zag butt joint for plate thickness of 12mm each and indicate all the dimensions clearly. |  |
| b) | Interprete the weld symbols shown in fig.2 as per IS 813 system. |  |
| c) | Sketch a column splicing to join bottom column of ISWB 500 @ 95.2 Kg/m to top column of ISWB 400@ 66.7 Kg/m. |  |
| Q.No.4. | Answer the following Questions: |  |
| a) | Using ISA 110 110 x 10mm, draw to scale Isometric at ‘O’ and Orthographic views (FV, TV, SV) of structure shown in fig.3. Use point ‘O’ as origin to draw Isometric view. | (8) |
| b) | Make Bill of Material (BOM) for above structure. | (4) |
| Q.No.5. | Answer the following Questions: |  |
| a) | Draw the orthographic view and Developed view of the portion of pipeline whose pictorial view is shown in fig.4. (single line diagram) | (8) |
| b) | Sketch Turn Buckel hanger and U-bolt as pipe supports. | (4) |
|  | ### OR |  |
| b) | Draw to scale the following sections: i) ISMC 175 @ 19.1 Kg/m ii) ISJT 100 @ 5.0 Kg/m | (4) |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Workout the weight of following: i) ISA 6040 x 5.0mm of 7532mm length ii) Steel plate of 840 x 921x12mm (l x b x t) | …….2/- |
|  | -2- |  |
| b) | How are weld represented on drawing? Explain. |  |
| c) | Briefly write the procedure to prepare fabrication drawing from design drawing. |  |
| d) | Sketch a typical shoe joint. |  |
| e) | Sketch a copper corrugated expansion joint. |  |