## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mechanical Engg./A

Subject: Machine Drawing(ME301) [Rev]

# Time Duration: 4 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give the conventional representation of the following: i) Wood ii) Concrete iii) Glass |  |
| b) | Give the conventional representation of the following: i) External Threads ii) Holes on linear pitch iii) Helical tension spring |  |
| c) | Show the single line representation of the following pipe fittings: - Coupling ii) Plug iii) Lateral |  |
| d) | Draw the symbols for the following: i) Double ‘Ú’ butt weld ii) Single ‘V’ butt weld iii) Single ‘J’ butt weld |  |
| e) | Represent the welded joint of 2 shafts with equal diameter welded end to end with a square butt weld and convex contour at site. |  |
| f) | Indicate the roughness symbol & roughness value for the following roughness grade numbers: i) N3 ii) N10 iii) N12 |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 5 = 10 |
| a) | Draw the conventional representation of the following sections: i) Full section ii) Half section. |  |
| b) | Draw the double line representation of a pipe line showing the following pipe fittings: - Tee ii) Cross iii) Reducer |  |
| c) | Represent the socket and spigot pipe joint with a neat sketch. |  |
| Q.No.3. | Draw neat proportionate free hand sketches: (any 2) | 2 x 5 = 10 |
| a) | V belt pulley |  |
| b) | Flat belt pulley with arms |  |
| c) | Muff coupling |  |
| Q.No.4. | Fig.(1) show the details of a footstep bearing. Draw the following views of the assembly: i) Full sectional front view ii) Top view. | (15) |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.5. | Fig.(2) shows the assembly of a socket and spigot joint. Draw the full sectional details of all the parts along with its side view. | (15) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 5 = 10 |
| a) | Using suitable welds, fabricate the components shown in fig.3. |  |
| b) | Explain the meaning of the symbols at a, b, c, d, e & f for the fig.4. |  |
| c) | Indicate the production symbols from (i) to (viii) as given on the component shown in fig.(5). Show the two orthographic views of the sheave of an eccentric. Redraw the vies and incorporate the following details: i) Cylindrical surface of 200mm to be machined and roughness to be indicated by  ii) Face on the left hand side to be machined and roughness to be indicated by  iii) Tolerance of H7 on 80mm iv) Roughness on the inner surface of hole of 80mm to be indicated by  v) Tolerance of K7 on the width of the keyway vi) Axis of hole of 80mm to be indicated as datum A vii) Face on the left hand side to be perpendicular to datum A with 0.1mm viii) Tolerance of e8 on 190mm |  |