## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Civil Engg./CT/FT&EE

Subject: Materials & Structures(4232)/(CE402) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain stress-strain relationship with respect to mild steel subjected to tensile force. |  |
| b) | Define Longitudinal strain, Lateral strain and Poisson’s ratio. |  |
| c) | What is moment of Inertia and radius of gyration? |  |
| d) | What are the assumptions made in theory of simple bending? |  |
| e) | Show that stress due to instantaneous stress is twice that of gradual loads. |  |
| f) | Define temperature stress and temperature strain. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A steel bar of length 600mm, has a diameter of 30mm for a length of 180mm, a diameter of 20mm for a length of 260mm and a diameter of 35mm over its remaining length. Determine the stresses in each section and elongation of the rod when it is subjected to a pull of 40KN. Take E= 2 x 105 N/mm2. |  |
| b) | A reinforced concrete column is 400mm x 400mm and is provided with 8 bars of 22mm diameter. Find the safe axial load on the column if the maximum stress in concrete is limited to 4N/mm2. Take Es= 18Ec. |  |
| c) | A steel rod is 25m long at a temperature of 20°C. Find the free expansion of the rod when the temperature is raised to 65°C. Also, find the temperature stress when the expansion of the rod is prevented. = 12 x 10-6 per° C and E = 2 x 105 N/mm2 |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Find the moment of Inertia of inverted T section about horizontal centroidal axis and about its bottom most fibre. Refer fig.1. |  |
| b) | Find the moment of inertia of section about centroid X–X axis and about top most fibre. Refer fig.2. |  |
| c) | Find the centre of gravity of the composite solid. Refer fig.3. students of Revised scheme) | (Only for |
| c) | A steel bar, 300mm long, 50mm wide and 12mm thick is subjected to an axial pull of 84KN. Determine change in volume in the bar if E = 2 x 105 N/mm2 and 1/m = 0.32. students of Old scheme) | (Only for |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Cantilever AB is fixed at A and free at B. It is loaded as showing in fig.4. Draw SFD and BMD. |  |
| b) | Simply supported beam of span 8m is loaded with three point loads as shown in fig 5. Draw SFD and BMD. -2- | ……..2/- |
| c) | An overhang beam ABC has an overhang BC of span 2m as shown in fig.6. It is loaded as shown in fig.6. Draw SFD and BMD. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A beam of rectangular section of 450mm depth and MI = 27536 cm4 about its axis of bending. Find the maximum permissible span for the beam if simply supported at the ends. It has to carry a uniformly distributed load of 25000 N/m without exceeding bending stress of 120N/mm2. |  |
| b) | A rectangular timber beam of 3.6m span has a uniformly distributed load of 2KN/m. Find the dimension of the cross-section. Take depth of the section as twice the width of the section and maximum bending stress as 7N/mm2. |  |
| c) | A circular pipe of external diameter 100mm and internal diameter 80mm is used as simply supported beam. The span of the beam is 4m. Find the safe concentrated load that the beam can carry at the mid span if the permissible stress in the beam is 120N/mm2. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A simply supported beam has cross section as T section with top flange 125mm x 25mm, web thickness as 25mm and overall depth 200mm. It is subjected to shear force of 150KN. Calculate the maximum shear stress at the neutral axis. |  |
| b) | An ‘I’ section with top and bottom flange of 150mm x 20mm, with web thickness 10mm and overall depth of 290mm is subjected to a shear force of 100KN. Determine the shear stress distribution for the beam section. |  |
| c) | A unknown weight falls through 10mm on a collar rigidly attached to the lower end of a vertical bar 3m long and 600mm2 in section. If the maximum instantaneous extension is 2mm, what is the corresponding stress and value of unknown weight? |  |