## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mechanical Engg./A

Subject: Theory of Machines(4039)/(ME501) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Mention any three inversions of double slider crank chain. Draw a neat sketch of any one of them. |  |
| b) | How are followers classified? |  |
| c) | Draw the crank effort diagram for a single cylinder four stroke cycle I.C. engine. |  |
| d) | Explain hunting of a governor. |  |
| e) | With reference to gears i) Circular pitch ii) Addendum iii) Pressure angle. |  |
| f) | What is a dynamometer? Mention any three types of dynamometers. |  |
| g) | What are the effects of vibration in machines? |  |
| h) | Mention any three applications of gyroscope. (only for students of old scheme) |  |
| Q.No.2. | Answer the following Questions: |  |
| a) | A disc cam rotating clockwise at a uniform speed of 200 rpm is required to give motion to a reciprocating roller follower as below: i) Follower to move outward through 30mm during 120° of cam rotation with uniform velocity ii) Follower to dwell for the next 60° of the cam rotation iii) Follower to return to its original position during the next 100° of the cam rotation with equal and uniform acceleration and retardation iv) Follower to dwell for the rest of the cam rotation The minimum radius of the cam is 50mm. The diameter of the roller is 20mm. The axis of the follower passes through the cam shaft axis. Draw the profile of the cam. Calculate the maximum velocity and acceleration of the follower during its return stroke. | (9) |
| b) | Define the terms coefficient of fluctuation of speed and coefficient of fluctuation of energy as related to a flywheel. | (3) |
|  | -2- | ……2/- |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The spindle of a screw jack has single start square thread with outside diameter of 45mm and a pitch of 10mm. The spindle moves in a fixed nut. The load is carried on a swivel spindle head but not free to rotate. The bearing surface of the swivel head has a mean diameter of 60mm. The coefficient of friction between the nut and the screw is 0.12 and that between the swivel head and the spindle is 0.1. Calculate the load that can be raised by an effort of 200N applied at the end of the lever of effective length 350mm. Also determine the efficiency of the screw jack. |  |
| b) | A thrust bearing has a number of contact surfaces, 200mm external diameter and 150mm internal diameter. The coefficient of friction is 0.08 and the total axial thrust is 22.5KN. Assuming uniform pressure intensity of 0.35 MN/m2, determine i) The number of collars required ii) The power absorbed in friction at 450 rpm. |  |
| c) | Explain with the help of a neat sketch the working principle of Inertia governor. |  |
| d) | For the slider-crank mechanism shown in fig.1, determine the velocity and acceleration of slider B by approximate analytical method. (only for students of old scheme) |  |
| Q.No.4. | Answer the following Questions: |  |
| a) | A flat belt, 240mm wide and 14mm thick, transmits power from one pulley to another. The ratio of driving tensions is 2.1, maximum permissible stress in the belt material is 2.2 MPa and mass density of the belt material is 1Mg/m3. Determine the speed at which the power transmitted is maximum and the maximum power transmitted. | (6) |
|  | ### OR |  |
| a) | A leather belt transmits 8 KW of power from a pulley 1100mm in diameter running at 200rpm. The angle of lap is 160° and the coefficient of friction between the belt and the pulley is 0.25. The safe working stress in the belt is 2.2 N/mm2. The thickness of the belt is 8mm and the density of leather is 0.001 g/mm3. Find the width of the belt considering centrifugal tension. | (6) |
| b) | Draw the involute tooth profile of spur gear for the following data showing two teeth. Pitch circle diameter = 240mm Number of teeth = 20 Pressure angle = 20° | (6) |
|  | -3- | ….3/- |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The band brake shown in fig.2 is applied to a drum of 220mm diameter. The coefficient of friction between the band and the drum is 0.25. Find the torque applied on the drum due to the pull of 100N at the end of the lever for clockwise rotation of the drum. |  |
| b) | With the help of a neat sketch, explain the construction and working of internal expanding shoe brake. |  |
| c) | With the help of a neat sketch, explain the construction and working of Bevis-Gibson flash light torsion dynamometer. |  |
| d) | The dimensions of the various links of a mechanism shown in fig.3 are as below: OA = 80mm, AC = CB = 120mm, CD=200mm If the crank OA rotates at 150 rpm in the anticlockwise direction, find the velocity of slider D for the given configuration. (only for students of old scheme) |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with the help of a neat sketch the construction and working of Whitworth quick return motion mechanism. |  |
| b) | Three masses M1, M2 and M3 with magnitudes 10 Kg, 9 Kg and 16 Kg revolve in the same plane at radii 100mm, 125mm and 50 mm respectively. The angular position of M2 and M3 from M1 are 60° and 135° respectively in the anti clockwise direction. Determine the magnitude and position of mass M4 placed at a radius of 150mm to balance the system. |  |
| c) | Explain the terms: i) Longitudinal vibration ii) Transverse vibration iii) Torsional vibration iv) Forced vibration. |  |