## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Civil Engg.

Subject: Soil Mechanics(CE505) [Rev]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define (i) Water content (ii) Porosity (iii) Degree of saturation. |  |
| b) | Write a short note on plasticity chart. |  |
| c) | What is bearing capacity of soil? Enlist limiting values of any two types of soil. |  |
| d) | Write down the procedure to find field dry density. |  |
| e) | Draw diagram showing soil as three phase system. |  |
| f) | What is area ratio and recovery ratio of sample? Give its significance. |  |
| g) | Differentiate between compaction and consolidation. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain grain size distribution curve. |  |
| b) | What are consistency limits? Explain how shrinkage limit is determined in laboratory. |  |
| c) | State Darcy’s law and enlist factors affecting permeability. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Distinguish between standard proctor and modified proctor test. |  |
| b) | A soil sample is partially saturated. Its natural moisture content was found to be 22% and bulk density 2 g/cm3. If the specific gravity of solid particle is 2.65 and density of water be taken as 1g/cm3, find degree of saturation and void ratio. |  |
| c) | Give comparison between laboratory & field tests of soil. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List down the limitations of plate load test. |  |
| b) | Explain the principle of Direct shear test. State its advantages. |  |
| c) | Explain consultant head permeability test with aid of neat sketch. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a brief note on types of settlements in soils. |  |
| b) | Explain in detail tri-axial test. |  |
| c) | What is the object of soil exploration? Also mention how location, depth & spacing of exploration decided. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Soil reinforcement with geotextile |  |
| b) | Bore-hole log |  |
| c) | Soil grouting |  |
| d) | Importance of effective stress in Engineering problem |  |

***********