## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Drives(4115)/(EE601) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is multimotor drive? Give one example of such drive. |  |
| b) | List any six types of enclosures for motor as per IS 4722. |  |
| c) | State the types of drive motor used for the application of lift. Justify your answer. |  |
| d) | Draw and briefly explain the typical heating and cooling curve of a drive motor. |  |
| e) | Define steady state stability of a drive. Also state other types of stabilities in drive. |  |
| f) | State the factors on which speed of three phase Induction motor depends. |  |
| g) | State one application/use of each of the following: - Time delay relay ii) Overload relay - Push button |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram, explain the four quadrant operation of a motor driving a hoist load. |  |
| b) | Draw the joint speed torque characteristics of electric motor and driven unit. Explain it in brief. What is its significance? |  |
| c) | State the different types of loads based on variation of Load torque with time to be driven by electric motor. Also state at least two examples of each load. |  |
| d) | Give the classification of Electric drive & list down factors governing the selection of motor. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Classify electric breaking used for drive. Which of the braking is most efficient, semi efficient and inefficient giving reasons for your choice. |  |
| b) | With the help of neat diagram, explain counter current braking for dc shunt motor in brief. Also draw the speed v/s torque characteristics. |  |
| c) | Define braking. State the advantages and limitations of electric braking. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with the help of a circuit diagram the cascade connection of speed control for 3 phase slip ring Induction motor. |  |
| b) | What is the significance of voltage to frequency control in three phase Induction motor? |  |
| c) | What is the necessity of Load equalization? How can it be achieved? Explain in brief with the help of speed v/s time curve & torque v/s time curve. |  |
|  | -2- | …..2/- |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the effects of environmental factors on the performance and ratings of drive motor. |  |
| b) | List out the different classes of duty cycle of electric motor. State suitable examples of any four classes of duty of your choice. |  |
| c) | Draw a neat labelled control circuit diagram indicating reversal of direction of rotation of motor by using direct method and explain it in brief. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Using a neat labelled power circuit diagram explain in brief remote control operation of three phase Induction motor. |  |
| b) | With a neat control diagram explain the sequential operation of three motors A, B & C such that motor A starts first followed by B and then C. (use sequence control using TDR) |  |
| c) | State the type of motors used for following applications and justify you choice: i) Pumps ii) Machine tools. |  |
| d) | Write a short note on drive used in textile mills. (only for students of old scheme) |  |

***********