## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Shipbuilding Engg.

Subject: Computer Applications In Ship Design(SH723)

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any three: | 3 x 5 = 15 |
| --- | --- | --- |
| a) | Sectional area curve |  |
| b) | GRT and NRT |  |
| c) | Buoyancy curve |  |
| d) | Intact stability calculations |  |
| e) | Detailed design of Hull block |  |
| Q.No.2. | Explain the procedure to obtain the freeboard for a particular ship. What are freeboard corrections? | (12) |
|  | ### OR |  |
| Q.No.2. | A bulk carrier has a depth of 6m and a draught of 4.5m. The total enclosed volume is 20,000m3 of which 80% is used for cargo. Calculate the gross and net tonnages. What is the net tonnage of the ship if it is now decided to carry 19 passengers in cabin? | (12) |
| Q.No.3. | Explain the procedure to develop lines plan of the hull form from the sectional area curve and the load waterline. | (12) |
| Q.No.4. | How is the sectional area curve of a basis ship is modified for a new position of LCB of the vessel and for a new prismatic co-efficient? Explain the sequence. | (12) |
| Q.No.5. | Explain how the light ship weight of ship determined during weight curve generation. | (12) |
|  | ### OR |  |
| Q.No.5. | The mean values for weight and buoyancy as measured at centres between each of six stations are as follows: Station; Weight (tonnes/m); Buoyancy (tonnes/m). ½; 43.51; 17.99. 1½; 85.62; 69.64. 2½; 47.98; 85.96. 3½; 41.31; 73.97. 4½; 34.65; 46.98. 5½; 52.11; 10.66. Determine the bending moment values for the ship. | (12) |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the various forms of data input in ship design software. |  |
| b) | How is damaged stability determined in ship design software? |  |
| c) | Explain the importance of Hull component library in Hull block generation. |  |

***********