## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Materials & Processes(4238) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Classify plain carbon steels based on carbon content and give their applications. |  |
| b) | What are the common alloying elements in steel? Explain their effects. |  |
| c) | Differentiate between grey and white cast iron. |  |
| d) | Explain the tensile test performed on steels. What is the formula used for obtaining ultimate tensile stress? |  |
| e) | What is the purpose of Annealing and how it is done? |  |
| f) | Define corrosion of welds and explain the various types of corrosion in welds. |  |
| g) | What is Age hardening? |  |
| h) | Distinguish between cold and hot working. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is meant by ‘Eutectic’, ‘hypoeutectic’ and ‘hypereutectic’ solution on iron-carbon equilibrium diagram. |  |
| b) | Explain the effect of heat treatment and atmospheric exposure on mechanical properties of steel. |  |
| c) | Describe the ‘Tensile test’ and explain how ultimate tensile stress, yield stress and percentage elongation is obtained. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain any two methods used to reduce welding stresses. |  |
| b) | Explain the metallurgical changes which occur due to absorption of gases by welds. |  |
| c) | Discuss the different types of tests used for identifying the metals and alloys. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain need for Heat treatment. What are the factors affecting good heat treatment? |  |
| b) | Discuss tempering and normalising processes for steel. |  |
| c) | Explain the concept of epitaxial growth and competitive growth in weld metal solidification. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the role of quenching media in Heat treatment. Give the names of various quenching mediums. |  |
| b) | Explain the mechanism of work hardening. State its practical applications. |  |
| c) | Explain the procedure of Forging and Extrusion with a neat sketch. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | High speed steels |  |
| b) | Spheroidal graphite iron |  |
| c) | Heat effected zone (HAZ) |  |
| d) | Types of stainless steels |  |
| e) | Composite materials |  |

***********