## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Electronics Engg./EC/EI/COMP/MED/E&EE

Subject: Microprocessors-I(8085) [4142] [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the terms opcode and operand. |  |
| b) | Define subroutines. Give the instructions to implement subroutine. |  |
| c) | Give the functions of i) Program counter ii) Stack pointer. |  |
| d) | Compare serial and parallel communication. |  |
| e) | Differentiate between Vectored and Non vectored interrupts. |  |
| f) | Specify the contents of registers and flag status after the execution of the following instructions: MVI A, 5EH ADI A2H MOV C, A HLT |  |
| Q.No.2. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Give a neat diagram of the Bus structure of IC 8085. |  |
| b) | Explain the Microprocessor initiated operations. |  |
| c) | Explain the entire register structure of IC 8085. |  |
| d) | Give the schematic diagram for the generation of the read/write control signals for memory and I/O. |  |
| e) | State the functions of the following pins: i) INTR and INTA ii) HOLD and HLDA. |  |
| Q.No.3. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain any two Rotate instructions. |  |
| b) | Give thee meaning and indicate the addressing mode of the following instructions: i) MOV B,C ii) CPI 39H iii) STA 3900H iv) CMA |  |
| c) | Draw the timing diagram showing the contents of the address and data buses and relevant control signals for the instructions MVI B, 77H. |  |
| d) | Explain the steps involved in fetching, decoding and executing any 8085 instruction. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write an ALP to count and display continuously in Hexadecimal from FFH to 00H in a system with 0.5µsec clock period. Use register ‘C’ to setup a one millisecond (1ms) delay between each count and display the numbers between the two counts. |  |
| b) | Assume that one data byte is stored at memory location 3500H. Write a program which tests the 5th bit of the data. Write FFH into location 3501H if the 5th bit is ‘0’ and write 00H in location 3501H if the 5th bit is ‘1’. |  |
| c) | 10 data bytes are stored in consecutive memory locations starting from XX00H. Write a program to transfer the entire block of data in a reverse order in consecutive locations starting from XX50H. |  |
|  | -2- | …….2/- |
| Q.No.5. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Explain any one Advanced subordinate technique. |  |
| b) | Give the details of the following interrupts: TRAP and RST7.5. |  |
| c) | Explain how to interface a display (7 segment display) to Microprocessor. |  |
| d) | Explain how masking of interrupts is done. Give the instruction which does the same. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Memory Mapping |  |
| b) | Indexing |  |
| c) | Conditional CALL and RET instruction |  |
| d) | Peripheral initiated operations |  |
| e) | Addressing modes |  |

***********