## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Mechanical Engg.

Subject: Machine Design Practice(4045)/(ME601) [Old/Rev]

# Time Duration: 4 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is standardisation? What are the benefits of standardisation? |  |
| b) | What is proof stress? Explain how the proof stress is determined. |  |
| c) | Why is the taper provided on a cotter? A single taper is preferred over double taper cotter, why? |  |
| d) | Write an equation for determination of shaft diameter on the basis of Rigidity. |  |
| e) | What are the requirements of good coupling? |  |
| f) | Define the following terms used in screw threads: i) Core diameter ii) Pitch iii) Lead. |  |
| g) | What are the functions of spring? List different types of springs. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The socket and spigot cotter joint shown in Fig.1 is subjected to an axial load of 100KN. The material used for spigot end, socket end and the cotter is steel having tensile yield strength of 330MPa. Assume factor of safety of 4. Determine the dimensions marked ‘d’, ‘d2’, ‘t’, ‘b’ and ‘a’ in the fig.1. |  |
| b) | A knuckle joint is subjected to an axial pull of 150KN. Design the following parts of the knuckle joint: i) Rod ii) Eye end of rod iii) Knuckle pin. Assume the following permissible stress for all the parts. Tensile stress t = 70MPa, Shear stress s = 60MPa, Crushing stress c = 150Mpa |  |
| c) | An offset link subjected to a force of 30KN is shown in fig.2. It is made up of grey cast iron having ultimate stress (tensile) ut = 300 N/mm2. Determine the dimensions of the cross section of the link. Assume factor of safety ‘3’. |  |
|  | -2- | ….2/- |
| Q.No.3. | Answer any one of the following Questions: | (12) |
| a) | Design a shaft to transmit power from an electric motor to a lathe head stock through a pulley by means of a belt drive. The pulley weighs 200N and is located at 300mm from the centre of the bearing. The diameter of the pulley is 200mm and the maximum power transmitted is 1.5KW at 200 rpm. The angle of lap of the belt is 180 and coefficient of friction between the belt and the pulley is 0.3. The shock and fatigue factors for bending and twisting are 1.5 and 2.0 respectively, The allowable shear stress for the shaft material may be taken as 35MPa. Refer Fig.3. |  |
| b) | i) Find the diameter of a solid steel shaft to transmit 20KW at 200 rpm. The ultimate shear stress may be taken as 360MPa and a factor of safety as ‘8’. If a hollow shaft is used in place of the solid shaft, find the inside and outside diameter of the hollow shaft. The ratio of outside diameter to inside diameter may be taken as ‘2’. ii) Design a Muff coupling used for connecting two shafts transmitting 30KW at 400 r.p.m. The material for the shaft and key is plain carbon steel for which the maximum shear stress and crushing stress are 100MPa and 200MPa respectively. The Muff is of cast Iron for which maximum shear stress is 30MPa. Use factor of safety as ‘2’. | (6) (6) |
| Q.No.4. | Answer the following Questions: |  |
| a) | Design a rectangular key for transmitting 1200 N-m torque to the pulley mounted on it. The permissible stresses for the shaft as well as for key are shear stress of 50MPa and crushing stress of 95MPa. Assume an overload factor of 1.25. | (4) |
| b) | A bushed pin type flexible coupling is used to connect a pump to a motor shaft. The details are as given below. Power transmitted 32 KN at 960 rpm; Overall torque is 20% more than mean torque; the allowable shear stress for the shaft and pin material is 80 N/mm2; the permissible bearing pressure for rubber bushes is 1.5 N/mm2. There are 4 pins and placed on P.C.D which is 4 times the diameter of the shaft. Determine i) Shaft diameter ii) Diameter of pins iii) Dimensions of bushes. | (8) |
|  | ### OR |  |
| b) | A power screw having double start square threads of 25mm nominal diameter and 5mm pitch is acted upon by an axial load of 10KN. The outer and inner diameter of screw collar are 50mm and 20mm respectively. The coefficient of thread friction and collar friction may be assumed as 0.2 and 0.15 respectively. Assuming uniform wear condition at the collar and allowable bearing pressure of 5.8 N/mm2, find i) The torque required to rotate the screw ii) the stresses in the screw iii) the number of threads of nut in engagement with the screw iv) the length of the lever required to raise the screw, if the effort applied is 300N. | (8) |
|  | -3- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | For supporting the travelling crane in a workshop the brackets are fixed on steel column as shown in fig.4. The maximum load that comes on the bracket is 12KN acting vertically at a distance of 400mm from the face of the column. The vertical face of the bracket is secured to column by four bolts, in two rows (two in each row) at a distance of 50mm from the lower edge of the bracket. Determine the size of the bolts if the permissible value of the tensile stress for the bolt material is 84MPa. |  |
| b) | A steel plate 100mm wide and 10mm thick is joined with another steel plate by means of a single transverse and double parallel fillet welds as shown in fig.5. The strength of the welded joint should be equal to the strength of the plates to be welded. The permissible tensile and shear stress for the weld material and plates are 70N/mm2 and 50N/mm2 respectively. Find he length of each parallel fillet weld. Assume the tensile force acting on the plate as static. |  |
| c) | i) What do you mean by fatigue failure? Draw S-N curve showing endurance limit. ii) What is Wahl’s correction factor? Why is it taken in design of spring? |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A rail wagon of mass 20 tonnes is moving with a velocity of 2 m/s. It is brought to rest by two buffers with springs of 300mm diameter. The maximum deflection of springs is 250mm. The allowable shear stress in the spring material is 600MPa. Find the following: i) Diameter of wire ii) Number of active coils iii) Free length of the spring. Assume G = 85 KN/mm2. |  |
| b) | A locomotive semi elliptical laminated spring has an overall length of 1m and sustains a load of 70KN at its centre. The spring has 3 full length leaves and 15 graduated leaves with a central band of 100mm width. All the leaves are to be stressed to 400MPa, when fully loaded. The ratio of the total spring depth to that of the width is 2. E = 210KN/mm2. Determine: i) Width of the leaves ii) Thickness of the leaves iii) Initial gap that should be provided between the full length and graduated leaves before the band load is applied. |  |
| c) | Write short notes on: - Applications of Sliding contact and Roller contact bearings - Selection of Ball bearing from manufacturer’s catalogue | …..4/- |