## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

April, 2018 Examinations

Programme: Automobile Engg.

Subject: Automobile Chassis-II(4062) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do you mean by tractive effort and tractive effort curves? |  |
| b) | State the function of leaf springs in Automobile. |  |
| c) | How is the stability of vehicles on turns & levelled roads achieved? |  |
| d) | What are different car body styles? Briefly explain the meaning of Sedan car. |  |
| e) | What is Hot dip galvanization? |  |
| f) | What is the function of king pin in steering system? Define the term ‘toe’ in steering system. |  |
| g) | What is the basic difference between a fluid flywheel and torque convertor? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A fully loaded car weighing 2.032 tonnes has transmission efficiency 88% in top speed, road resistance 23 kg per tonne and air resistance o.00843V2, where ‘V’ is the speed of the car. Calculate i) b.h.p required for a top speed of 144 km/hr. ii) the acceleration at 48 km/hr assuming the torque at 18 km/hr in top gear is 25% more than at 144 km/hr. |  |
| b) | What is overall gear ratio? Explain the term pitching & swaging. |  |
| c) | Explain the terms: i) Acceleration ii) Gradiability iii) Draw bar pull in relation to automobile performance. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define Camber, Castor & Slip Angle. What should be the approximate amount of the following in a car: camber, king pin inclination, included angle, castor & toe in? |  |
| b) | Why is pinion usually placed tilted to rack in the rack and pinion type steering gear? State advantages of rack and pinion steering gear. |  |
| c) | Define ‘turning circle’ and state its approximate value for any car and bus. What is the purpose of ball joints in steering linkage? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the working of an Epicyclic Gear Train. |  |
| b) | Sketch and describe the working of a semi-automatic gear box. |  |
| c) | What is the necessity of gearbox at all in automobile when engine speed can be varied by acceleration? Explain the term ‘Overdrive’. |  |
|  | -2- | ….2/- |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How is side thrust countered in a vehicle? Sketch a leaf spring joint and state the materials used for it. |  |
| b) | With a neat sketch explain the working of a vertical guide type suspension system. |  |
| c) | What is the purpose of independent suspension? State advantages of air suspension over conventional suspension. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat sketch explain the luxury seat mechanism of an automobile. |  |
| b) | Explain with sketch the layout of different compartments of an automobile. |  |
| c) | Explain the various rust inhibiting techniques to prevent corrosion. |  |

***********