## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mechanical Engg.

Subject: Power Engineering-II(4041) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Psychrometric Charts, Steam Tables and Molliers

Chart is permissible.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | With neat sketches, explain different types of nozzles. |  |
| b) | What do you mean by ‘compounding’ in steam turbines? Why is it required? |  |
| c) | What are the effects of air leakage in steam condensers? How can air leakage be minimized? |  |
| d) | State 3 major differences between 2 stroke and 4 stroke cycle engines. |  |
| e) | State the functions of condenser in ignition system of S.I. engines. |  |
| f) | Define (i) Relative humidity (ii) Wet bulb temperature (iii) Dew point temperature. |  |
| g) | State the advantages of gas turbine over internal combustion engine. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What do you understand by governing of I.C. engines? State the different methods of governing of I.C. engines and describe briefly any one method. |  |
| b) | With the help of neat sketch explain the construction and working of a jet condenser. |  |
| c) | With the help of suitable sketches, explain the pressure-velocity compounding of an impulse turbine. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In a laboratory experiment, the following observations were noted during the test of a four stroke Diesel engine: Area of indicator diagram = 420mm2 Length of indicator diagram = 62 mm Spring number = 1.1 bar/mm Diameter of piston = 100mm Length of stroke = 150 mm Engine speed = 450 r.p.m. Determine (i) Indicated mean effective pressure (ii) Indicated power. |  |
| b) | A steam jet enters the row of blades with a velocity of 375 m/s at an angle of 20º with the direction of motion of the moving blades. If the blade speed is 165 m/s, find the suitable inlet and outlet blade angles assuming that there is no thrust on the blades. The velocity of steam passing over the blades is reduced by 15%. Also determine power developed by the turbine per Kg of steam flowing over the blades per second. |  |
| c) | The vacuum efficiency of a condenser is 96%. The temperature of condensate is 40ºC. If the barometer reads 752 mm of Hg, find the vacuum gauge reading of the condenser. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the water cooling system in I.C. engines. |  |
| b) | With the help of a neat sketch, explain the valve timing diagram for a four stroke cycle diesel engine. |  |
| c) | A Diesel engine has a compression ratio of 20 and cut off takes place at 5% of the stroke. Find the air standard efficiency. Assume  =1.4. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch, explain the working of a simple vapour absorption refrigeration system. |  |
| b) | What are the different sources of heat load on airconditioned space? Explain the procedure of cooling load estimation of a given space. |  |
| c) | With the help of a neat sketch, explain the working of an open cycle gas turbine. State its advantages over closed cycle gas turbine. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | State the classification of steam turbines. Give examples. |  |
| b) | Distinguish between jet condensers and surface condensers. Give 4 points. |  |
| c) | State the advantages of gas turbine over steam turbine. |  |
| d) | Dry saturated steam at a pressure of 15 bar enters a nozzle and is discharged at a pressure of 1.5 bar. Find the final velocity of steam, when the initial velocity of steam is negligible. |  |

***********