## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mechanical Engg.

Subject: Energy Conversion(ME405) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | How are air compressors classified? |  |
| b) | Draw P-V and T-S diagrams for Otto cycle and list the sequence of processes. |  |
| c) | Find the break thermal efficiency of an engine which consumes 7.35 Kg of fuel in 20 minutes, while developing a break power of 92KW. The calorific value of fuel is 44800 KJ/Kg. |  |
| d) | State the various factors to be considered while selecting a site for a nuclear power plant. |  |
| e) | Discuss the potential of wind energy. |  |
| f) | What is a ‘biogas’? |  |
| g) | How the large diesel engines are started by using compressed air? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the pressure control mechanism used in compressors. |  |
| b) | Explain following terms with respect to air compressors: i) Compression ratio ii) Free air delivered iii) Volumetric efficiency. |  |
| c) | Describe the construction and working of axial flow compressors. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare and contrast SI engine and CI engine. (any six points) |  |
| b) | State the functions of following parts of an engine: i) Piston ii) Connecting rod iii) Crank shaft iv) Inlet valve. |  |
| c) | Describe the construction and working of two stroke petrol engine. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss various engine cooling methods. |  |
| b) | The following particulars refer to a two stroke oil engine: Bore =20cm diameter Stroke = 30cm Speed = 350 rpm Indicated mean effective pressure = 275 KN/m2 Net brake load = 610N Diameter of Brake Drum = 1m Oil consumption = 4.25 Kg/hr Calorific value of oil = 44000 KJ/Kg Determine i) Indicated power ii) Brake power iii) Mechanical efficiency iv) Brake thermal efficiency. |  |
| c) | State salient features of coal fired thermal power plant. (any four) |  |
|  |  | …….2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is a Solar still? Discuss its construction and working. |  |
| b) | Discuss the construction and working of horizontal type of wind mill. |  |
| c) | Describe any one type of Biogas plant giving a neat sketch. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Inter cooling and after cooling in compressors |  |
| b) | Steam generator |  |
| c) | Solar cooker |  |
| d) | Wind data |  |

***********