## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mining Engg.

Subject: Underground Mining Machinery(MN403)/(4294) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are the advantages of mechanisation? |  |
| b) | Explain the specifications of Jack hammer. |  |
| c) | Write short note on superelevation. |  |
| d) | Mention the different parts and accessories of a cage. |  |
| e) | How a round stand rope is different from flattened strand rope? |  |
| f) | Mention the essentials of a good haulage track. (For students of old scheme only) |  |
| f) | Differentiate between Mechanical & Hydraulic transmission systems. (For students of revised scheme only) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and operation of LHD used in mines. |  |
|  | For students of revised scheme only: |  |
| b) | What are the types of maintenance techniques? What features does Maintenance Information Systems consist of? |  |
| c) | With a schematic sketch explain the constructional details of Tunnel Boring Machine. |  |
|  | For students of old scheme only: |  |
| b) | Explain the terms i) Tractive effort ii) Coefficient of adhesion. |  |
| c) | Describe the construction and operation of a Road Header. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe an endless haulage system. How are the tubs attached to the haulage rope? |  |
| b) | What are the various safety measures incorporated in Diesel locomotives? Gives a brief note on Exhaust conditioner. |  |
| c) | The data given below are available in respect a direct rope haulage at the top of an incline. Length of incline - 500m Gradient - 1 in 7 Number of loaded tubs per set - 6 Weight of each loaded tub -1.5T Speed of rope - 8 Km/hr Weight of rope -1.63 Kg/m coefficient of tub friction -1/50 coefficient of rope friction - 1/10 Calculate the minimum size of the electric motor in KW required to work the haulage assuming a gear efficiency of 70% and motor efficiency of 90%. Assume data, if required. |  |
|  |  | …..2/- |
|  | -2- |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the various drum winding profiles (shapes) used in winding? How do they perform with regard to torque? |  |
| b) | Explain the ground mounted Koepe winding system with a neat sketch. |  |
| c) | Mention the safety measures incorporated in the winding system. In what way caliper brakes are superior to post brakes? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the construction of a belt? Calculate the carrying capacity for coal of a 900mm width belt conveyor when the cross section of the load is of 0.074 sq.m. and the belt moves at a speed of 125 m per minute. |  |
| b) | Explain the purpose and function of i) safety hook ii) keps. |  |
| c) | Explain the construction and operation of a reliance capel. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Differentiate between DERDS and SERDS. |  |
| b) | Explain the operation of i) Back catch ii) Drop warwick. |  |
| c) | Mention specific features of any one type of Pit-top layout. |  |
| d) | How do you prevent deterioration of wire ropes in mines? |  |

***********