## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Repair Engineering(SH602)/(5334) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) Q.No.1 is compulsory. Answer any 5 from the remaining questions.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short notes on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Nature of work carried out by a ship repair yard |  |
| b) | Temporary repairs |  |
| c) | Cathodic protection |  |
| d) | Hatch cover survey |  |
| e) | Painting scheme of ship’s hull |  |
| f) | Composite boiler |  |
| g) | Survey of Life saving appliances |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the ship lift docking system and state the advantages and disadvantages of this system. |  |
| b) | List out the defects, which are commonly found during survey of hull structure. Briefly explain any two defects with the repair methods used. |  |
| c) | What preparations need to be made prior to docking a vessel? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the procedure for the removal of propeller and tail end shaft. What inspections are carried out on propeller? |  |
| b) | Discuss inspection/survey and testing of cargo cranes. |  |
| c) | Explain survey and repairs of Anchor chain cables. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the methods of surface preparations and the standards of cleaning achieved by these methods. |  |
| b) | Explain how the crankcase inspection is carried out in diesel engines. |  |
| c) | Describe maintenance of a two stage air compressor along with its accessories. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How are damaged FRP hull laminates repaired? |  |
| b) | With respect to office procedure and costing, explain the terms: i) Quotation ii) Work order iii) Direct and Indirect costs. |  |
| c) | Briefly explain the weighment and servicing of CO2 bottles on board ship. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the procedure of overhauling a fuel pump. |  |
| b) | What are the general safety regulations to be followed in a ship repair yard and on board repair ships? |  |
| c) | Describe maintenance of a shell and plate type heat exchanger used for fresh water cooling. |  |
| Q.No.7. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Area of maximum wastage |  |
| b) | Load line marks |  |
| c) | History sheet for machinery maintenance |  |
| d) | Safety for Hot work |  |

***********