## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Microprocessor Based Instrumentation(5114) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the functions of bus interface unit and execution unit in 8086. |  |
| b) | What is the function of Bus High Enable (BHE) signal of 8086? |  |
| c) | State the function of carry flag and auxiliary carry flag in 8051. |  |
| d) | What is the function of RST pin in 8051? |  |
| e) | Illustrate with suitable instructions how bits of PSW register can be used for selection of register banks. |  |
| f) | What do RI and TI flags indicate in 8051? |  |
| g) | Explain the difference between XCH instruction and XCHD instruction. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare features of 8051, 8031, 8751 and 8748 microcontrollers. |  |
| b) | Describe the need for segment registers and index registers in 8086. |  |
| c) | - What is the maximum memory size that can be addressed by 8086? - What is the maximum clock frequency used in 8086? - Differentiate between 8086 and 8088 microprocessors. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With suitable block diagram, describe the architecture of 8051 microcontroller. |  |
| b) | List any three SFRs of 8051 and explain their functions. |  |
| c) | Describe the various modes of timer operation in 8051. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the steps in executing an interrupt in 8051. |  |
| b) | Explain functions of various bits of TCON register. |  |
| c) | Describe the steps to transfer data serially using 8051. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the meaning and state the type of instruction and addressing mode for each of the following instructions: - MOV A @ R1 ii) ORL A, #51H ii) INC 30H |  |
| b) | List down and explain the various conditional jumps used by 8051. |  |
| c) | Write two instructions each, which use the following addressing modes: i) immediate ii) register iii) direct. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a program to multiply 6 by 4 and store the result in internal RAM location 65H. |  |
| b) | Write a program to toggle the bits of port 1 with a delay which depends on a value loaded in R0. |  |
| c) | Write a program to find the number of zeroes in register R2. |  |

***********