## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Drawing(FE302)/(4234) [Old/Rev]

# Time Duration: 4 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

4) Use of Structural Engineer’s Handbook SP:6(1) & IS 800

is permissible.

5) All dimensions in the figures are in mm.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw a roller support for pipeline. What are its special features? |  |
| b) | Sketch a typical ridge joint. |  |
| c) | Redraw the following rolled steel sections to scale by referring appropriate tables: - ISLB - 200 (ii) ISA 9060 x 10mm |  |
| d) | Illustrate the difference between shop and site weld. |  |
| e) | Draw two views and show how column is connected to beams using riveted joints. |  |
| f) | What is Axial pitch, Radial pitch and Diagonal pitch, in a zig-zag double riveted lap joint? |  |
| g) | Draw one view each, of the following trusses: - King post truss (ii) Queen post truss |  |
| Q.No.2. | Draw the gusset plate drawing at the ends ‘M’ & ‘N’ of the member MN shown in fig.1. What is cut size of this member? Assume: - All members are ISA 6060 x 8mm thick - Snap headed rivets of 16mm are used - Gusset plate thickness is 10mm | (12) |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Illustrate the following structural connections: - Bearing plates for beam ends - A splice joint connecting two beams - A plate girder |  |
| b) | - What is the relationship between throat thickness and size of a fillet weld? Explain with a sketch. - A bracket is to be welded all round as shown in fig.2. Redraw and indicate appropriate weld symbols. Weld all around using 8mm weld size and site welding. |  |
| c) | Draw neat sketches of the following: - Rag Bolt (ii) Loop Bolt | …….2/- |
|  | -2- |  |
| Q.No.4. | Answer the following Questions: |  |
| a) | The top frame of a structure shown in fig.3 is made of ISA 5050 x 4.0mm. Corner legs are ISLB 100. Interconnecting members are Flats 50 x 6.0mm. Draw orthographic views (FV, TV, LHSV) of the structure when viewing in direction ‘x’ of arrow shown. | (8) |
| b) | Make of bill of material (B.O.M) for the structure shown in fig.3. | (4) |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Draw isometric single line diagram of pipeline whose orthographic views are given in fig.4. Use point ‘O’ as origin. |  |
| b) | Draw developed view of pipeline shown in fig.4. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | A 10mm plate is to be connected to another similar plate with a single strap single riveted lap joint. Work out the various dimensions and draw proportionate, to scale, two views of this connection. |  |
| b) | Calculate the weight of 5m length of ISMC300. |  |
| c) | Shown how a 4 member welded connection of a truss using gusset plate is made. Write down the steps involved. |  |
| d) | Draw a copper corrugated expansion joint. |  |
| e) | Refer fig.5. Workout the exact length of central vertical member ‘AB’ by assuming suitable clearances, allowances used for truss fabrication. |  |
|  | All angles used ISA 50 x 50 x 6. | …..3/- |