## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electrical Engg.

Subject: Generation & Transmission(4114) [Old]

# Time Duration: 3 Hrs.                                           Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State and explain any three advantages of Grid system. |  |
| b) | Why is a moderator necessary in a nuclear power plant? |  |
| c) | Give any six specifications for procurement of stand-by diesel generator sets. |  |
| d) | Explain in brief ACSR conductors. |  |
| e) | Compare RCC supports with wooden and steel supports. |  |
| f) | Define the terms voltage regulation and transmission efficiency with respect to transmission lines. |  |
| g) | What do you mean by stringing of conductors? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the factors to be considered while selecting the number of units for a power plant and preparing the operating schedule. |  |
| b) | With a neat diagram explain in brief working of a hydroelectric power plant. |  |
| c) | A generating station has the following daily load cycle. Time (hrs) 0-6 6-10 0-12 12-16 16-20 20-24 Load (MW) 40 50 60 50 70 40 Draw the load curve and find (i) maximum demand (ii) units generated per day (iii) average load (iv) load factor. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a short note on electrical power generation using tidal energy. |  |
| b) | Discuss the methods of starting Diesel Generator sets in brief. |  |
| c) | Compare pin insulators with suspension insulators. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | List the advantages and disadvantages of HVDC transmission system. |  |
| b) | What do you mean by string efficiency? Discuss the methods to improve string efficiency. |  |
| c) | Explain how the characteristics of medium transmission line can be determined by nominal  method. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is corona? Explain in brief methods to reduce corona effect. | ……2/- |

-2-

| b) | Explain the effect of variation of transmission voltage and load p.f. on the performance of short and medium transmission lines. |  |
| --- | --- | --- |
| c) | A single phase overhead transmission line delivers 1100KW at 33KV at 0.8 p.f. lagging. The total resistance and inductive reactance of the line are 10 and 15 respectively. Determine (i) sending end voltage (ii) sending end power factor (iii) transmission efficiency. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the vibrations observed in power conductors. Explain their effect and any one method used to damp out these vibrations. |  |
| b) | Write a short on: (i) Sag template (ii) Capacitance of transmission line. |  |
| c) | A transmission line has a span of 150m between level supports. The conductor has a cross-sectional area of 2cm2. The tension in the conductor is 2000Kg. If the specific gravity of the conductor material is 9.9 gm/cm3 and wind pressure is 1.5 Kg/m length, calculate the sag. Also calculate vertical sag. |  |

***********