## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Ship Resistance & Propulsion(SH601)/(4339) [Old/ [Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Write short note on any five: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Kinematic similarity |  |
| b) | Froude’s Number |  |
| c) | Propeller Blade sections |  |
| d) | Wake |  |
| e) | Controllable pitch propeller |  |
| f) | Spade Rudder |  |
| Q.No.2. | Answer any one of the following Questions: | (12) |
| a) | Explain how the resistance of a ship can be determined from model tests. |  |
| b) | A ship of length 116m, breadth 18.85m, draft 7.20m and displacement 8450 tonnes has the following values of effective power. |  |
|  | Speed of ship (knots); 15.5; 16.5; 17.5; 18.5; 19.5. Effective Power (KW); 2300; 2860; 3553; 4390; 5425. |  |
|  | Determine for a geometrically similar ship of displacement 9350 tonnes, the effective power at a speed of 17.5 knots. |  |
|  | Given (i) Kinematic viscosity = 1.115 x 10-6 m2/s (ii) Wetted surface are = 2550 m2 of ship of length 116m. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A ship whose wetted surface area is 5150 m2 travels at 15 knots. Calculate the frictional resistance if Froude’s constant f = 0.422 and power of speed n = 1.825. |  |
| b) | Explain the purpose and procedure of model testing. |  |
| c) | Explain the propulsion terms: (i) Thrust Power (ii) Delivered Power |  |
| Q.No.4. | Answer any one of the following Questions: | (12) |
| a) | A propeller rotates at 2 rev/s with a speed of advance of 12 knots and a real slip of 0.30. The torque absorbed by the propeller is 250 KNm and the thrust delivered is 300 KN. Calculate: (i) The pitch of the propeller (ii) The thrust power (iii) The Delivered Power. |  |
| b) | What is a screw propeller? Draw a typical propeller drawing and explain the geometry. |  |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | Explain the advantages and disadvantages of a water lubricated stern gear. |  |
| b) | Explain any one of the below mentioned special propeller types with salient features, advantages and disadvantages: - Vertical axis propeller (ii) Super cavitating propeller |  |
| Q.No.6. | Answer the following Questions: | 2 x 6 = 12 |
| a) | When does a ship need to be provided with a reduction and reversing gear box? |  |
| b) | Describe briefly the different types of rudder. (only for students of revised scheme) |  |
| b) | Describe the construction and function of ‘A’ Bracket with neat sketch. (only for students of old scheme) |  |

***********