## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Civil Engg.

Subject: Soil Mechanics(CE505) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Draw detailed soil phase diagram for soil. |  |
| b) | Define Relative density. How it is used in describing cohesionless soil? |  |
| c) | Define Darcy’s law, Coefficient of permeability and Seepage velocity. |  |
| d) | Write any three points of comparison between Direct shear test and Tri-axial test. |  |
| e) | Define i) Compression Index ii) Coefficient of consolidation iii) Degree of consolidation. |  |
| f) | Write the purpose and scope of soil exploration. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define void ratio, porosity, degree of saturation, water content, specific gravity of soil grains and unit weight. |  |
| b) | With aid of neat sketch explain plasticity chart. |  |
| c) | Explain the behaviour of cohesive soils with change in water content. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | In a falling head permeameter test on a silty clay sample, the following results were obtained. Sampling length -12mm, sample diameter - 80mm, initial head - 1200mm, final head - 400mm, time for fall in head - 6 minutes, stand pipe diameter - 4mm Find the coefficient of permeability of soil. |  |
| b) | With aid of neat sketch explain Mohr-Coulomb failure theory. |  |
| c) | Explain drainage condition of soil sample for measurement of shear strength. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the importance of effective stress in engineering problems. |  |
| b) | Explain with neat sketch shear stress and strain curve. |  |
| c) | Explain the procedure for standard penetration test to calculate bearing capacity of soil. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the job of an embankment supervisor in relation to compaction. |  |
| b) | Explain the factors affecting compaction. |  |
| c) | Explain the methods of soil exploration. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Factors affecting permeability |  |
| b) | Engineering classification of soil |  |
| c) | Soil nailing |  |
| d) | Soil Reinforcement with geotextile |  |
| e) | Disturbed, undisturbed and representative sample |  |
| f) | Factors affecting bearing capacity of soil |  |

***********