## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Civil Engg./CT/MNE

Subject: Surveying-I(4001) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the general principles of Survey and explain how they all useful in survey. |  |
| b) | Draw a neat sketch of following instruments and mention their specific use in measuring distances: - Ranging rod ii) Peg iii) Arrow |  |
| c) | What are the factors to be considered in the selection of survey stations for chain triangulation? |  |
| d) | Differentiate between ‘Open’ and ‘Closed’ traversing. Sketch and give examples for each. |  |
| e) | Explain the method of entering the readings in a field book of chain and compass survey. |  |
| f) | State the reasons for “Local Attraction”. How can you detect it from the readings? |  |
| g) | What is “Simple Levelling”? When and where it is used? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How will you measures distance on a sloping ground by the method of stepping? Sketch and explain. |  |
| b) | What is meant by “Direct Ranging”? When it is done? How it is done? |  |
| c) | Explain the method of taking off-sets for locating existing ground features such as trees, electric pole, well, etc. on a plot. Explain with neat sketch. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | How will you overcome the following obstacles which are obstructing during measurement: - A hill is obstructing the vision during chaining - A building is obstructing both vision and chaining |  |
| b) | Explain the method of setting perpendicular to a survey line using optical square. How it is done at site? Explain. |  |
| c) | How the area is calculated for an irregular plot from the observations recorded in a chain and cross staff survey. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) Differentiate between Whole Circle Bearing and Reduced Bearing. ii) Convert the following WCB’s to Reduces Bearings: - 6530 2) 25510 3) 33640 |  |
| b) | Sketch the cross section of a Prismatic compass and name the component parts. |  |
| c) | The bearings of the sides of a Traverse ABCDE are as follows: Sr. No.; LINE; FORE BEARING; BACK BEARING. 1; AB; 9715; 27715. 2; BC; 120; 1920. 3; CD; 27130; 9130. 4; DE; 18915; 0915. 5; EA; 12445; 30445. Calculate the interior angles of the traverse. | ……..2/- |
|  | -2- |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Define the following: 1. Level surface 2. Level line 3. Horizontal line 4. Vertical line 5. Axis of bubble tube 6. Line of collimation |  |
| b) | i) State the special features of following instruments: 1) Dumpy Level 2) Tilting Level 3) Auto Set Level ii) What are the classifications of Levelling? Where they are used? |  |
| c) | The following consecutive readings were taken with a Dumpy Level: 3.860, 3.345, 2.930, 0.850, 3.795, 2.640, 1.540, 1.930, 0.865 and 0.665 The level was shifted after the fifth and eighth readings. The first reading was taken on a Bench mark whose R.L. was 150.250. Calculate R.L’s of all the points and the difference of level between the first and last points. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) State the uses of following instruments: i) Planimeter ii) Abney Level iii) Box Sextant ii) Write short notes on ‘Bench marks’ and its types. |  |
| b) | What are the classification of Levelling and explain the specific use of each type? |  |
| c) | What do you understand by “temporary adjustment of a dumpy level”? Mention the different stages of its performance. |  |

***********