## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Materials & Processes(FE404) [Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is chrome plating? Give two examples of its use. |  |
| b) | Give applications where Rubber is used as an engineering material. |  |
| c) | Explain why Grey Cast Iron is used in majority of castings. |  |
| d) | What is Hydrogen embrittlement and cracking? |  |
| e) | Which are the factors affecting heat treatment of steel? |  |
| f) | What is ‘allotropy’ as used in study of pure Iron? Which are its allotropic forms? |  |
| g) | What is the effect of grain size on the mechanical properties of plain carbon steel? |  |
| h) | What are the reasons for alloying in steels? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Briefly describe the various types of plain carbon steels giving their %C content, properties and uses. |  |
| b) | Draw a neat sketch of a blast furnace and explain the various sections involved. |  |
| c) | What are Substitutional and Interstitial solid solutions? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write a note of various alloying elements used in steel and their effects. |  |
| b) | Discuss the Heat Affected Zone (HAZ) and its significance in welding. |  |
| c) | What is the need for stress relief in welding and briefly explain the methods used in relieving the stress? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | By showing the temperature range on a section of an Iron-carbon equilibrium diagram, explain the process of hardening of steel with steps involved. |  |
| b) | Draw and explain the T-T-T curves used in Heat treatment of steel. |  |
| c) | Describe the process of Al-Zn or Al-Cu alloys age hardening. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the differences in cold working and hot working processes? |  |
| b) | Write a note on Cathodic and Anodic protection of steel. |  |
| c) | Discuss any one process of mechanical cleaning of steel for surface protection. |  |
| Q.No.6. | Write short notes on any three: | 3 x 4 = 12 |
| a) | Composite materials |  |
| b) | Forging process |  |
| c) | Tempering |  |
| d) | Spheroidal Graphite Iron |  |
| e) | Stainless steel |  |

***********