## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Shipbuilding Engg.

Subject: Marine Regulations(SH611) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What is a Port State? |  |
| b) | What is a Classification Society? |  |
| c) | Define “Freeboard length’’ of a ship. |  |
| d) | What are the SOLAS requirements regarding ballast tank coatings? |  |
| e) | What is meant by “Clean ballast” in oil tankers? |  |
| f) | What is meant by “associated piping” in chemical tankers? |  |
| g) | What is SECA area as per MARPOL Annex – VI? |  |
| h) | What is meant by ‘Sewage’ in ships? |  |
| i) | What are the full forms of IBC Code and IMDG code? |  |
| Q.No.2. | What is IMO? What is the structure of IMO? Explain the functions of MSC and MEPC. | (12) |
|  | OR |  |
| Q.No.2. | Who is the Flag State Administration in India? What does Part IX of Indian Merchant Shipping Act deal with? How are marine regulations implemented in India? | (12) |
| Q.No.3. | Name the items, including documentation, that are to be surveyed/verified during an initial Loadline survey of a ship. | (12) |
|  | Explain in detail the inspection/survey procedures for any four of these items. |  |
|  | OR |  |
| Q.No.3. | Explain the following terms: | (12) |
|  | i) Depth for Freeboard ii) Freeboard deck iii) Superstructure iv) Flush–deck ship v) Deck line vi) Loadline marks vii) Seasonal freebards marks |  |
| Q.No.4. | What are the ‘sub-division’ requirements as per SOLAS regulations for all passenger ships and cargo ships of 80m length and upwards? Explain the same in terms of: ‘A’, ‘R’, ‘pi’, ‘si’, ‘As’, ‘Ap’, and ‘Al’ | (12) |
|  | OR |  |
| Q.No.4. | Write down atleast six of the general requirements of SOLAS regulations for ship’s steering gear. Explain the time limits for operation of steering gear in normal condition and during emergency steering. | (12) |
| Q.No.5. | What are the requirements of SOLAS regulations for bilge pumping arrangement of Passenger ships and cargo ships? | (12) |
|  | OR |  |
| Q.No.5. | What are the types of fire detection systems and fixed fire-fighting systems used in ships? Accommodation spaces, Machinery spaces, Cargo area of dry cargo ships and Cargo area of an oil tanker? | (12) |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Accidental oil outflow parameter from oil tankers are expressed by the following: | (12) |
|  | OM ≤ 0.015 for C ≤ 200,000 m3 |  |
|  | where OM = (0.40 Ms + 0.60 MB)/C |  |
|  | and OMB = 0.7 OMB (0) + 0.3 OMB (2.5) |  |
|  | Define the following: |  |
|  | C, OM, OMS, OMB, OMB (0) and OMB (2.5) |  |
|  | How are OMS, OMB(0) and OMB (2.5) calculated? |  |
|  | OR |  |
| Q.No.6. | What are the hazards of ship recycling? How does the International Convention for the Safe and Environmentally Sound Recycling Of Ships’ address the concerns of ship– recycling? | (12) |

***********