## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Civil Engg./CT

Subject: Analysis of Structures(CE504)/(4026) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Define limit of eccentricity and find the same for circular column of uniform diameter. |  |
| b) | Determine normal and tangential stress on a 45 plane for a member 100x100 in section subjected to pull of 16 KN. |  |
| c) | Define Fixed End Moments and draw BMD for a fixed beam with a UDL throughout the span. |  |
| d) | Define distribution factor and carry over factor. |  |
| e) | Define Buckling load, a short and a long column. |  |
| f) | Briefly explain procedure of determination of Moment of Resistance of a Flitched beam. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A Flitched Beam consists of a wooden beam 120 x 240mm size with a steel plate of 120mm wide and 10mm thick connected at top and bottom of wooden beam symmetrically. Given stress in steel 75N/mm2, find stress is wood and Moment of Resistance of Beam. Take Es=16Ew |  |
| b) | A hollow circular column of 150mm outer diameter and 20mm thick carries a vertical load of 160KN at an eccentricity of 60mm. Determine maximum and minimum stresses produced. |  |
| c) | A Bridge Pier of 1250 x 2000mm in size carries a vertical load of 1000KN at an eccentricity of 500mm from xx axis and 400mm to the right of yy axis. Determine stresses at all corners of column. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A beam is subjected to longitudinal bending stress of 85N/mm2 tensile and a transverse shear stress of 40N/mm2. Find magnitude and direction of principal stresses and maximum shear stress. |  |
| b) | A column is subjected to normal stresses of 100N/mm2 tensile and 40N/mm2 compressive in two mutually perpendicular planes. Find magnitude and direction of resultant stress of Mohr’s stress circle method. |  |
| c) | A cantilever beam of 2m span carries a point load of 4KN at 1m and 2KN at 2m from fixed end. Take EI = 16 x 1012 N-mm2. Find maximum deflection in beam. |  |
| Q.No.4. | Answer any one of the following Questions: | (12) |
| a) | A simply supported beam of 6m span carries a UDL of 30KN/m for the full span and point loads of 20KN each at a distance of 2m and 4m from left end. Determine deflection under point loads and maximum deflection. Take EI = 18 x 1012 N-mm2. Use Macaulay’s method. |  |
| b) | A fixed beam of 7m span carries a UDL of 40KN/m for a length of 4m from left support and also a point load of 30KN at 2m from right support. Find Fixed End Moments and sketch SFD and BMD. |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.5. | Answer any one of the following Questions: | (12) |
| a) | A continues beam ABCD consists of AB=5m, BC=5m and CD=4m lengths. Support A is fixed and D is a simple support, while B and C are middle simple supports. Span AB and CD carry UDL of 30KN/m throughout, while BC span carries point load of 40KN at centre. Using Moment Distribution method, analyse the beam and draw SFD and BMD. Assume uniform EI throughout. |  |
| b) | A portal frame ABCD has beam BC=6m while column AB=CD=4m. Supports A and D are fixed. Flexural rigidity(EI) of columns is 2EI while beam is 1.6 EI. Beam BC carries UDL of 40KN/m throughout. Using Moment Distribution method, analyse the frame and sketch BMD and SFD. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A rolled steel I section consists of flange plate of 200 x 15mm connected to both flanges of column. Height of column is 6m with both supports fixed. Determine safe load using Euler’s formula. Take E = 2 x 105 N/mm2 and factor of safety as 2.5. Properties of I Section: Ixx = 28083 x 104mm4 Iyy = 2728 x 104mm4 |  |
| b) | A hollow circular column has inner diameter 0.8 times its outer diameter. It carries an axial load of 400KN. Using factor of safety as 4, find sizes of column. Height of column is 5m with one end hinged and other fixed. Use Euler’s formula. E = 1.2 x 105 N/mm2. |  |
| c) | A rectangular column of 150 x 250mm in size is of 2m long with both supports hinged. Find critical load using Rankin’s formula. Take crushing stress c = 550 N/mm2 and Rankine’s constant  = 1/1600. |  |

***********