## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Food Technology

Subject: Food Engineering-I(FT304)/(4265) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any ten of the following Questions: | 10x1.5 =15 |
| --- | --- | --- |
| a) | Give an expression for heat transfer by conduction through a two-layered composite flat wall. |  |
| b) | Distinguished between the terms emissivity and absorptivity. |  |
| c) | What is a film heat exchanger? Give one application. |  |
| d) | Name three different types of tubular heat exchangers. |  |
| e) | State one problem encountered in heat transfer to gases, and how this can be overcome. |  |
| f) | Distinguish between humidity and relative humidity of a given sample of air. |  |
| g) | Find the RH of air, saturated at 25ºC after it is heated at 40ºC. |  |
| h) | Name three continuous dryers. |  |
| i) | Explain the principle of fluidization. |  |
| j) | Explain the significance of equilibrium moisture content. |  |
| k) | What is the main difference between the unit operations of dehydration and concentration? Give one example of equipment used for each of these operations. |  |
| l) | Name and define one unit of refrigeration. |  |
| m) | Name two ways frozen foods deteriorate on storage and state how this can be minimized. |  |
| n) | What is meant by “stripping of flavour”? How is this done? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the concept of thermal resistance. Hence define heat transfer coefficient and derive an expression for overall heat transfer coefficient for conduction of heat through pipes. |  |
| b) | Draw a diagram of a steam-heated tubular heat exchanger and label all parts. Saturated steam at 100ºC is condensing on the shell side of a tubular heat exchanger. The cold liquid enters the tube at 30ºC and leaves at 70ºC. Calculate arithmetic and log mean temperature differences. |  |
| c) | Give an account of mechanically aided heat exchangers. Why are they required and what are their uses? With a neat sketch explain their main feature. |  |
| Q.No.3. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | State Stefan Boltzmann Law of Radiation for a black body. Hence write an expression for radiation from any body. Clearly name all terms therein. |  |
| b) | With the help of suitable graphs, compare counter and parallel flow in a double–pipe heat exchanger. |  |
| c) | With respect to plate heat exchangers, briefly state their characteristics, uses and advantages and disadvantages in the food industry. |  |
| d) | List humidity measuring instruments. Explain the construction and use of any one of these instruments. |  |
| e) | Explain the principle and applications of cooling by evaporation. | ……2/- |
|  | -2- |  |
| f) | The composition of a material is: Water 64%, Protein 16%, Fat 10%, Carbohydrate 8% and Ash 2%. Express the composition on dry basis. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain, with the help of suitable graphs, the drying behaviour of food materials. What factors affect the drying rate? |  |
| b) | (i) Suggest a dryer for the following and give reasons for your choice: Milk, instant coffee, mashed potato, diced vegetable. |  |
|  | (ii) 1000 Kg of tomato juice of 8% solids is concentrated to 12% TSS. How much concentrate will be obtained? |  |
| c) | Explain the principle and construction of a freeze-dryer. For what type of foods is this dryer most suitable? |  |
| Q.No.5. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | With the help of sketch explain multi-effect evaporation. What are its advantages and applications? |  |
| b) | What are the advantages of vacuum concentration over atmospheric pressure process? Hence list its uses. How is vacuum produced? |  |
| c) | With the help of a labelled sketch explain, briefly the construction, working and uses of a flash evaporator. |  |
| d) | Briefly explain the components and working of a vapour compression refrigeration cycle. |  |
| e) | What are the properties desirable in an ideal refrigerant? Briefly compare any three common refrigerants. |  |
| f) | With the help of a thermodynamic graph explain the effect of changing the conditions at the expansion valve and evaporator. |  |
| Q.No.6. | Answer any four of the following Questions: | 4 x 3 = 12 |
| a) | Explain the construction, working and uses of a blast freezer. |  |
| b) | Compare the construction and uses of a plate freezer and an immersion freezer. |  |
| c) | Write an explanatory note on storage of frozen foods. |  |
| d) | Differentiate between quick freezing and slow freezing. How does the rate of freezing affect the quality of the final product? |  |
| e) | Define distillation. Explain the process and equipment of batch distillation and its uses in industry. |  |
| f) | Explain the process of steam distillation with the help of a diagram. State its uses. |  |

***********