## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mechanical Engg.

Subject: Automobile Engineering(5036) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the functions of the capacitor used in the battery ignition system. |  |
| b) | What is the role of a gear box in an automobile? How is the first gear different from the third? |  |
| c) | What is the use of a torque wrench? |  |
| d) | List down the features of a hermetically sealed compressor. |  |
| e) | State the important features of the monocoque construction. |  |
| f) | List the features of the throttle body injection system. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat sketch of an automobile frame and list down its salient features. |  |
| b) | Giving neat sketch explain how a power steering system operates. |  |
| c) | Explain giving a neat sketch how a torsion bar works. Compare a torsion bar to a helical spring. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State any four advantages of the multipoint fuel injection system over the carburetted system. |  |
| b) | Giving a neat schematic diagram, explain the construction and working of the multi-point fuel injection system. |  |
| c) | Write a note on: History and operation of automobiles. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with the help of a neat sketch the construction and operation of a constant mesh gear box. |  |
| b) | Giving neat sketch explain the construction and working of a fluid coupling. |  |
| c) | Write down the salient features of the Article 56 of the Motor Vehicle Act 1988 as applicable in India. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the construction and working of an overrunning clutch drive giving a neat sketch. |  |
| b) | Giving neat sketch explain the features of the lighting system employed in automobiles. |  |
| c) | Write a note on various compressors used in automobile air conditioners. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Hydraulic jacks |  |
| b) | Article 44 of the Motor Vehicle Act 1988 |  |
| c) | Allen keys |  |
| d) | Cut-out relay |  |
| e) | Transfer case |  |

***********