## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Fabrication Estimation & Costing(FE602)/(4242) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the purpose of estimating the cost of a product. |  |
| b) | What are the various functions of an Estimator? |  |
| c) | Explain the basic costing procedure in welding. |  |
| d) | Explain different sheet metal operations. |  |
| e) | State the advantages and disadvantages of Item Rate Contract. |  |
| f) | Define (i) Breach of contract with an example. (ii) Arbitration with an example. |  |
| g) | State the requirements of a valid contract. |  |
| Q.No.2. | Prepare a measurement sheet as well as Abstract sheet for a built up column with Batten plates as lateral supporting system for the following items. Assume suitable rates for the items. Refer Fig.1. | (12) |
|  | 1. Structural steel of rolled steel sections |  |
|  | 2. Structural steel of plates |  |
|  | 3. Anchor Bolts |  |
|  | 4. Welding |  |
| Q.No.3. | Answer the following Questions: |  |
| a) | Determine the time required to machine the job shown in Fig.2 of Brass from a 50mm diameter round stock of length 105mm. Assume suitable machining parameters. | (8) |
| b) | What are allowances? Explain any three allowances which are considered while determining machining time. | (4) |
|  | OR |  |
| b) | Explain different sheet metal joints. | (4) |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Estimate the cost of material of the job shown in Fig.2 made up of cast iron costing Rs.250/Kg and having specific weight of 7.2 gms/cc. | ……2/- |
|  | -2- |  |
| b) | Calculate the cost of welding per meter length for fillet welding 10mm thick steel plate using 4mm diameter electrodes. The electrode costs Rs.3/- each and the welder is paid Rs.500 per 8 hours shift. The electrical charge is Rs.20/unit. Assume suitable additional data. |  |
| c) | What is Break Even Theory? Explain the reasons for replacement of equipments. |  |
| d) | A machine was purchased at a cost of Rs.3,50,000/- and an establishment charge of Rs.7,000/- was incurred. It is expected to have a useful life of 9 years after which it may be sold at a scrap value of Rs.35,000/- but the demolition cost expected is Rs.3,800/-. Determine the cost of machine per hour. It has a motor of 4 HP and energy charge is Rs.20 per unit. (Only for students of old scheme) |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | A lathe worth Rs.6,50,000/- was purchased 4 years before with a useful life of 10 years and scrap value of Rs. 1,50,000/-. Calculate the depreciation by reducing balance method and find out the depreciation amount today. |  |
| b) | Explain the situations when Tender submitted becomes Informal Tender. |  |
| c) | Briefly explain the procedures involved in the execution of work in the Government departments like PWD. |  |
| d) | A machine is purchased for Rs.4,00,000/- with estimated life of 12 years having rate of interest on depreciation fund as 4% and scrap value of Rs.80,000/-. Calculate the rate of depreciation by sinking fund method. What is the depreciation amount at the end of five years? (Only for students of old scheme) |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Write short notes on any two: (i) Security Deposit and Earnest Money Deposit (ii) Lumpsum contract and Percentage Rate Contract (iii) Tender Notice |  |
| b) | Briefly explain: (i) Work Charged Establishment (ii) Schedule of Rates |  |
| c) | Explain (i) Contingencies Cost (ii) Technical sanction. |  |
| d) | Explain the various elements of costing. (Only for students of old scheme) |  |
|  |  | …….3/- |