## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Machines-I(EE302) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | For armature winding define i) Front pitch ii) Commutator pitch. |  |
| b) | Draw connection diagram for DC shunt generator and write related current and voltage equations. |  |
| c) | Why a DC motor is started using a starter? |  |
| d) | Write emf equation of a transformer. Hence define turns ratio of transformer. |  |
| e) | Draw a neatly labelled equivalent circuit of a transformer. |  |
| f) | List losses taking place in a transformer when it supplies to a load. Categorise them in constant and variable losses. |  |
| g) | State any three conditions to be satisfied to connect two 3 phase transformers in parallel. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain working of a simple loop D.C. generator. |  |
| b) | Explain the process of voltage built up in d.c. shunt generator using a neat diagram. |  |
| c) | With a neat diagram explain construction and working of any DC welding generator you know. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram, explain working of a three point starter. |  |
| b) | Draw and explain for DC series generator - Speed v/s armature current characteristics - Torque v/s armature current characteristics |  |
| c) | With a neat diagram, explain Ward-Leonard method of speed control of DC shunt motor. State its any two advantages. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Distinguish between core type and shell type transformers. Draw their schematic diagrams. |  |
| b) | With a neat diagram explain OFF load tap changer. |  |
| c) | In certain 50KVA transformer the number of turns of the primary and secondary winding are 834 and 58 respectively. If the primary is connected to 3300V supply, find - Secondary voltage - Primary and secondary current when the transformer is fully loaded. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is an ideal transformer? Draw its phasor diagram. Draw and explain phasor diagram of a practical transformer supplying an inductive load. |  |
| b) | With a neat diagram explain procedure for connecting two single phase transformers is parallel. | …..2/- |
|  | -2- |  |
| c) | In a 50 KVA transformer, Iron loss is 500W and full load copper loss is 800W. Find efficiency at full load and half full load at 0.8 p.f. lagging. |  |
| d) | A 10 KVA, 2000/400V single phase transformer has following data: R1=5Ω, X1=12Ω, R2=0.2Ω, X2=0.48Ω Find: - Secondary resistance referred to primary - Total resistance referred to secondary - Primary reactance referred to secondary - Total reactance referred to primary |  |
| Q.No.6. | Write short notes on any two: | 2 x 6 = 12 |
| a) | Open delta transformer |  |
| b) | Welding transformer |  |
| c) | ONAN method of transformer cooling |  |

***********