## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electrical Engg./E&EE

Subject: Substation Practices(EE602)/(5105) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | State the points to be taken into consideration while selecting a site for substation. |  |
| b) | What is a bus coupler? With a neat diagram explain its function. |  |
| c) | List the general specifications for procurement of CT. |  |
| d) | Draw the symbol for i) Earthing switch ii) Line trap. Explain the function of each. |  |
| e) | Explain in brief corridor type control and relay panels. |  |
| f) | What is a mimic diagram? What is its importance on control and relay panels? |  |
| g) g) | Explain in brief the function of any two main components of HVDC substation. (Only for old scheme students) State the desirable characteristics of tariff. (Only for revised scheme students) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain the construction of pole mounted substation. |  |
| b) | With a neat diagram explain single sectionalised busbar scheme. What are its merits? |  |
| c) | With neat diagrams explain wound type CT and bar type CT. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the single line diagram of 33/11KV substation and explain the function of lightening arrestor and isolator. |  |
| b) | Explain power factor tariff. |  |
| c) | State and explain causes of low power factor. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat diagram explain solid grounding. Draw phasor diagram for the same. |  |
| b) | i) Define Step potential and Touch potential. ii) State any three harmful effects of lightening. |  |
| c) | With a neat diagram explain how overhead ground wire provides protection against lightening. State its drawbacks. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the different factors affecting current carrying capacity of underground cables. |  |
| b) | With a neat sketch explain construction of XLPE cable. |  |
| c) | List any three methods of laying underground cables. Explain the draw in system in detail. What are its drawbacks? |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain in brief any three tests performed on transformer oil. | …….2/- |
|  | -2- |  |
| b) | Explain in brief the following methods of battery charging: - Constant voltage method - Constant current method - Trickle charging |  |
| c) | What are the precautions to be taken while carrying out maintenance work at substation? |  |

***********