## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mining Engg.

Subject: Opencast Mining Machinery(MN501)/(4299) [Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Enumerate causes of Bore hole deviation. |  |
| b) | Compare power shovel with dragline. |  |
| c) | Enumerate advantages and disadvantages of Dumper transportation system. |  |
| d) | Sketch and label dozer blade. |  |
| e) | State procedure for starting and stopping of pump. |  |
| f) | Explain types of fluid. |  |
| g) | Enumerate suitability conditions for different drilling methods. (only for students of revised scheme) |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain construction of front end loader. |  |
| b) | Explain surface miner. |  |
| c) | Describe walking mechanism of dragline. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain hydraulic feed mechanism in rotary drilling. |  |
| b) | Explain construction of hydraulic cylinder. |  |
| c) | Enumerate suitability conditions of Dumper and belt conveyor transportation system. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With sketch explain construction of mono-aerial ropeway. |  |
| b) | With sketch explain driving system of conveyor belt system. |  |
| c) | With sketch explain components of a Ripper. |  |
| Q.No.5. | Answer the following Questions: | 2 x 6 = 12 |
| a) | With sketch explain construction of centrifugal pump. |  |
|  | ### OR |  |
| a) | Explain submersible pump. |  |
| b) | A pump having efficiency of 75% is driven by a motor of 60 H.P. with a motor efficiency 90%. Find the quantity of water which can be pumped per minute to a total head of 50m. How many days it will take to dewater a flooded mine consisting 50000 m3 of water. Assume other conditions. |  |
| Q.No.5. | Write short notes: | 3 x 4 = 12 |
| a) | Lump Breaker |  |
| b) | Water hammer in Pump |  |
| c) | Comparison between shovel and Bucket wheel excavator |  |

***********