## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electronics Engg./EC/EI/COMP/E/E&EE/MED

Subject: Digital Electronics-I(4134) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Convert the following into decimal equivalent: (i) (100.10)2 (ii) (231)8 (iii) (EZ.OC)16 |  |
| b) | With reference to the digital logic family, define (i) Fan in (ii) fan out (iii) Noise immunity. |  |
| c) | State the two De Morgan’s theorems. |  |
| d) | Draw the symbol of an RS flip flop and write its truth table. |  |
| e) | State three differences between synchronous counter and asynchronous counter. |  |
| f) | State application of counters and shift registers. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Add the following decimal numbers using 8-bit 2’S complement arithmetic. - +43, + 37 (ii) +85, - 90 |  |
| b) | i) Convert 1011 to gray code |  |
|  | ii) Convert (27.1)8 to binary |  |
|  | iii) Convert (ED.2)16 to binary |  |
|  | iv) Convert (29)10 to binary |  |
| c) | Realize the following gates using NAND gates only: - NOT (ii) OR (iii) AND (iv) NOR |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Simply the following Boolean functions in SOP and POS forms f (A,B,C,D) = Σ (0,1,2,3,5,6,8,9,14,15) |  |
| b) | Draw the symbol and write the truth table of the following logic gates: AND, OR, NOT, NAND, NOR, EX-OR |  |
| c) | With the help of logic diagram and truth table explain the working of RS flip flop using NAND gates. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the logic diagram, symbol and truth table of a JK flip flop. Explain its working. How can it be converted to a T flip flop? |  |
| b) | Draw the diagram of a JK Master Slave and D flip flop and explain its working. |  |
| c) | Explain the working of a ring counter. What is its application? |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Design mod 8 asynchronous up counter. Explain its working and draw the waveforms. |  |
| b) | Draw the logic diagram of 4 bit serial–in serial-out Shift Register. Draw the waveforms and explain how data is shifted in and shifted out of this register. Consider the data to be 0110. |  |
| c) | Draw the circuit and explain the working of TTL-NAND gate with totempole output. |  |
|  |  | …..2/- |
|  | -2- |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw the circuit and explain the working of ECL NOR gate. |  |
| b) | Draw the circuit diagram of RTL using NOR gate and explain its working with help of truth table. |  |
| c) | Draw the circuit and explain the working of (i) CMOS Inverter (ii) CMOS NOR gate. |  |

***********