## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Automobile Engg.

Subject: Automobile Engines-I(4054) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Give a broad classification of IC engines. |  |
| b) | Define compression ratio. If the swept volume of a single cylinder SI engine is 98cc and the clearance volume is 12cc, calculate the compression ratio. |  |
| c) | What are the functions of i) Carburettor ii) Venturi iii) Throttle valve. |  |
| d) | During idling of an SI engine a rich air fuel mixture is needed. Why? |  |
| e) | Define Cetane number. How is it determined for an unknown fuel? |  |
| f) | Why does an IC engine need a cooling system? |  |
| g) | What is the function of a thermostat? Sketch a dole type thermostat. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is scavenging? With a neat labelled sketch explain the working of a crankcase scavenged engine. |  |
| b) | Describe i) Crankshaft ii) 2 types of mufflers, using neat sketches. |  |
| c) | Explain construction and functions of flywheel and vibration damper. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain for IC engine fuel what is Ignition quality and how this is rated. |  |
| b) | With the help of a neat schematic diagram, explain the working of an Electronic petrol injection system in an MPFI engine. List the various important sensors and actuators and state their function. |  |
| c) | Explain with neat sketch the construction and working of SU Electric fuel pump and list its advantages over the mechanical pump. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What is the function of radiator and its cap? List various types of cores for radiator. |  |
| b) | Explain with neat sketch the air injection system and one type of Airless injection for diesel engine. |  |
| c) | What is an injector of CI engine? Explain its construction and working with the help of a neat diagram. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch explain the working of wet sump lubrication system. |  |
| b) | Explain ignition delay in CI engine. How does it affect the knock? |  |
| c) | Explain detonation in SI engine and its effects. |  |
|  |  | …….2/- |

|  | -2- |  |
| --- | --- | --- |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the functions of lubrication? List all types of lubricants. |  |
| b) | Draw the performance curves for a petrol engine and comment on them. |  |
| c) | Explain the swinging field D.C. Dynamometer. |  |

***********