## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electrical Engg./E&EE

Subject: Electrical Machines-II(EE401)/A.C. Machines(4113)

[Old/Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | A slip ring induction motor runs at 290 r.p.m. at full load, synchronous speed = 300rpm when connected to 50Hz. Determine the number of poles and slip. |  |
| b) | Why is the air gap between rotor and stator of a 3 induction motor kept as short as possible? |  |
| c) | What do you mean by hunting of alternators? |  |
| d) | What is a synchronous capacitor? |  |
| e) | Single phase induction motor is not self starting. Explain why. Also list the various methods to make it starting. |  |
| f) | What is synchronous reactance of an alternator? |  |
| g) | How does the behavior of a synchronous motor differ from a 3 phase induction motor? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a Torque Slip curves briefly describe the relation between torque and slip for 3 induction motor. |  |
| b) | Explain the principle of working of 3 induction motor. What is meant by slip? |  |
| c) | A 50Hz, 8 pole induction motor has full load slip of 4%. The rotor resistance/phase = 0.01 ohm and standstill reactance/phase = 0.1 ohm. Find the ratio of maximum to full load torque and the speed at which the maximum torque occurs. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With a neat circuit diagram explain the working of DOL starter used for starting of a 3 induction motor. |  |
| b) | Explain briefly the power flow in the three phase Induction motor. |  |
| c) | Explain the equivalent circuit diagram of a three phase induction motor. |  |
| d) | Briefly explain concept of soft starter & its advantages. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain construction and working of capacitor start and capacitor run motor. |  |
| b) | Explain the term armature reaction. Explain armature reaction at lagging p.f. load in an alternator. |  |
| c) | Explain the advantages of stationary armature and rotating magnetic field for an alternator. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the conditions that must be satisfied for proper synchronization of alternators? Explain the effect of change of excitation on power factor. |  |
| b) | Briefly explain the effect on current and power factor under variable excitation & constant load in case of synchronous motor operation. | ……2/- |
|  | -2- |  |
| c) | Explain the construction and operation of hysteresis motor. |  |
| d) | Explain any one method of synchronizing of alternators. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw neatly vector diagram of a loaded alternator at unity p.f. , lagging p.f. and leading p.f. load. |  |
| b) | Give the classification of single phase motor based on methods of starting. Mention applications of each type. |  |
| c) | What do you mean by voltage regulation of an alternator? What are the various methods to calculate it? Explain any method in detail. |  |
| d) | Write a short note on ‘Repulsion motor’. |  |

***********