## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Mining Engg.

Subject: Mine Surveying(MN611) [Revised]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What do CMR 1957 say regarding appointment & qualifications of Mine Surveyor? |  |
| b) | Discuss the uniqueness of Mine surveying. |  |
| c) | Mention the applications of remote sensing. |  |
| d) | How do you determine mine boundary in open cast survey? |  |
| e) | What are the factors affecting stope surveying? |  |
| f) | Name the types of Mining plans & sections that need to be maintained in underground mines. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What precautions should be taken to guard against the surveying errors? |  |
| b) | Explain the methods of plotting plans & sections. |  |
| c) | i) Explain the principle of GIS. ii) What precautions are taken in shaft plumbing? |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | The following offsets were taken at 20m intervals from a survey line to an irregular boundary line: 3.55, 4.30, 5.85, 6.25, 7.20, 6.40, 6.8, 6.25, 4.20, 3.40m Calculate the area enclosed between the survey line, the irregular boundary line, and the first and last offsets, by: - The trapezoidal rule ii) Simpson’s rule. |  |
| b) | A coal heap on level ground measures 60m x 18m. The side slopes at 1.5 vertical to 1 horizontal and the average height of the heap is 3m. Calculate the quantity of coal in the heap assuming 1.10 cu.m of coal to one tonne. |  |
| c) | Explain the process of measuring coal stock on uneven ground. What precautions are taken during measurements? |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | What are the different types of faults? In a seam dipping at 1 in 8 one upthrow fault is met with. The fault hades at 30 and has thrown the seam up by 14.4m. Calculate the length of a drift rising at 1 in 4 driven to meet the seam on the other side of the fault. Assume that the direction and rate of dip of the seam remains unchanged on the other side of the fault. |  |
| b) | Explain with a neat sketch the stope surveying in cut and fill method. |  |
| c) | What are the different methods of correlation? Explain correlation by two shaft method. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the principle & components of EDM. |  |
| b) | What is total station? Explain the setting up & operation. | …….2/- |
|  | -2- |  |
| c) | i) Explain the components of GPS? ii) Write short notes on Bore Hole surveying. |  |
| Q.No.6. | Answer any three of the following Questions: | 3 x 4 = 12 |
| a) | Give the applications of cyrotheodolite in mines. Mention its limitations. |  |
| b) | How do you calculate the volume of box cut? |  |
| c) | Explain any one method of curve setting in underground mines. |  |
| d) | Explain any one method of reducing of plans. |  |

***********