## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electronics Engg./EC

Subject: Programmable Logic Controller(EN611) [Rev.]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the types of PLCs depending on the structure. |  |
| b) | Why are PLCs popular in the automation industry? |  |
| c) | Draw ladder diagrams to represent the following logic functions: - AND ii) OR iii) NOT |  |
| d) | Explain the status bits of a count-up counter. |  |
| e) | Explain the function of battery backed relays. |  |
| f) | List any three typical faults found in PLCs. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the different sections of memory used in a PLC. |  |
| b) | With the help of a diagram, explain sinking input and sourcing input and the difference between the two. |  |
| c) | Describe with the help of diagram: - Transistor output unit - Relay output unit |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | i) Explain the scan cycle with flowchart. ii) Explain the Masked Move instruction with an example. |  |
| b) | i) Draw the function block of any three arithmetic operations and describe them. ii) Write the Boolean expression representing the ladder diagram shown below. |  |
| c) | Develop a function block diagram and a ladder diagram for the following application. A pump is required to be activated to pump liquid into a tank when the START switch is closed and the level of liquid in the tank is below the required level and there is liquid in the reservoir from which the liquid is to be pumped. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw and explain the function block of a count down counter. |  |
| b) | Explain with an example of ladder diagram, cascading of timers. Also explain the need for cascading. | ……2/- |
|  | -2- |  |
| c) | i) Draw a ladder diagram for the following. A fan turns on 40seconds after a lamp is turned on. A STOP button turns off the lamp and the fan simultaneously. ii) Explain in brief internal relays used in PLC programming. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe a Master Control Relay and explain how these relays are used in the case of an emergency. |  |
| b) | Draw and explain the function block of a Bit Shift left instruction. |  |
| c) | i) Explain the Jump and Label instructions. ii) Draw the function block for the Equal to and the Greater than instructions and describe them. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Describe the steps involved in commissioning a PLC. |  |
| b) | Describe the criteria for selection of PLCs. |  |
| c) | i) Differentiate between a normal PLC and a safety PLC. ii) Write any three rules to be followed when writing ladder programs. |  |

***********