## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Fabrication Technology & Erection Engg.

Subject: Welding Technology-II(FE403)/(4240) [Old/Rev]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What type of power is used to weld Aluminium and their alloys by GTAW process? Why? |  |
| b) | Explain the need of gas preheater for CO2 gas shielding. |  |
| c) | List three important advantages of Ultrasonic welding. |  |
| c) | Explain heat balancing in Resistance Spot welding. |  |
| d) | Explain the different types of distortion in welded fabrication. |  |
| e) | Explain the difference between Repair welding and Maintenance welding. |  |
| f) | Explain the applicability of Electro-slag welding process. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch the principle of operation and working of GTAW process. |  |
| b) | Explain the different arc control mechanisms use in GMAW process. |  |
| c) | Explain the different types of weld backing used in SAW process. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Compare FCAW process with GMAW process. |  |
| b) | Explain with a neat sketch the principle of operation and working of Thermit welding process. |  |
| c) | Explain in detail the operating procedure of Electro-slag welding process with consumable guide tube system. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the different methods used to control distortion in welded fabrication. |  |
| b) | Explain the factors influencing the procedure to be adopted in maintenance welding. |  |
| c) | Explain with a neat sketch the principle of operation and working of Flash Butt resistance welding. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain stepwise the procedure involved to carry out a repair welding of a component. |  |
| b) | Explain the importance of Jigs and Fixtures in welding. Explain with neat sketch the working of a Quick acting clamp. |  |
| c) | Explain with a neat sketch the principle of operation and working of Laser welding process. |  |
| Q.No.6. | Write short notes on any four: | 4 x 3 = 12 |
| a) | Prevention of weld porosity in SAW |  |
| b) | Electrode holders in FCAW process |  |
| c) | Disadvantages of Electro-slag welding |  |
| d) | Resistance seam welding |  |
| e) | Residual stresses |  |
| f) | Welding Positioners |  |

***********