## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electronics Engg./EC

Subject: Television Engineering(5135) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks:  75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Compare the NTSC system with PAL in brief. |  |
| b) | Explain in brief how a Vidicon tube works. Draw the figure. |  |
| c) | Explain why AM and not FM is used for transmission of picture. Draw relevant diagrams. |  |
| d) | What is the use of AGC? Mention the types of AGC. |  |
| e) | Explain the function of colour burst. Where is it located in the video signal? |  |
| f) | What are weighting factors? Explain their necessity. |  |
| g) | Explain in brief any one fault associated with the vertical deflection circuit in a TV receiver. How is it rectified? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of neat sketch explain the working of a Plumbicon camera tube. |  |
| b) | Draw a neat sketch of a monochrome picture tube and label it. Briefly explain its working. |  |
| c) | Explain the working of a typical 5 element antenna used for TV reception. Give in brief the design steps involved in making such an antenna. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat block diagram and explain the electromagnetic deflection system employed in a TV receiver. |  |
| b) | Draw a neat block diagram of the sound section of a TV receiver and explain its working. |  |
| c) | Draw a neat sketch to illustrate the composite video signal and explain its various significant portions. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of suitable examples explain how compatibility is established between monochrome and colour TV system. |  |
| b) | Draw a neat diagram and explain the working of a Trinitron picture tube. |  |
| c) | With the help of a block diagram explain the working of a PAL-D colour receiver. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Draw a neat block diagram of a NTSC coder and explain its working. |  |
| b) | Explain the following: - Pin cushion adjustment - Bandwidth of the colour signal |  |
| c) | Explain the troubleshooting procedure for the following faults: - Picture normal but no signal - Sound ok but only a horizontal bright line in the center of the screen |  |
|  |  | ……2/- |
|  | -2- |  |
| Q.No.6. | Write notes on any four: | 4 x 3 = 12 |
| a) | Equalizing pulses |  |
| b) | Pincushion adjustment |  |
| c) | Frequency spectrum of TV channel |  |
| d) | DC reinsertion |  |
| e) | PIL picture tube |  |
| f) | Preemphasis |  |

***********