## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Electronics & Instrumentation Engg.

Subject: Machine Technology(4217) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | Explain the principle of pressure compounding used in steam turbines. |  |
| b) | Differentiate between Reed type valve and Plate type valve used in air compressors. |  |
| c) | Explain the term slip with respect to belts. |  |
| d) | State the function of Fusible plug. |  |
| e) | How does economiser improve efficiency of a steam boiler? |  |
| f) | What are the advantages of using Air vessels in reciprocating pumps? |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain Whitworths quick return mechanism. |  |
| b) | When is Oldham’s coupling used? Explain with a neat sketch its construction and working. |  |
| c) | Describe a Platform scale with the help of a neat sketch. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State law of belting. Explain cross belt drive with the help of a neat sketch. |  |
| b) | What do you understand by a compound gear train? Explain with the help of a neat sketch. |  |
| c) | With the help of a neat sketch, explain construction of Lancashire boiler. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the function of water level indicator. Explain its construction with the help of a neat sketch. |  |
| b) | With the help of a neat sketch explain the construction and working of an evaporative type of condenser. |  |
| c) | Describe an ejector type of a jet condenser with the help of a neat sketch. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | With the help of a neat sketch explain the construction and working of a velocity compounded steam turbine. |  |
| b) | Explain the construction and working of a vane type of air compressor with the help of a sketch. |  |
| c) | Describe the construction and working of a reciprocating air compressor. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the function of a foot valve in centrifugal pump. Explain the working of a centrifugal pump. |  |
| b) | Explain the construction and working of water ring vacuum pump. |  |
| c) | Define following terms: i) Kinematic link ii) Kinematic chain. |  |

************