## BOARD OF TECHNICAL EDUCATION

PORVORIM-GOA

November, 2016 Examinations

Programme: Automobile Engg.

Subject: Automobile Engines-II(4061) [Old]

# Time Duration: 3 Hrs.                                               Max. Marks: 75

Instructions: 1) All Questions are compulsory.

2) Figures to the right indicate full marks.

3) Assume suitable additional data if required.

| Q.No.1. | Answer any five of the following Questions: | 5 x 3 = 15 |
| --- | --- | --- |
| a) | What are DIN standards? |  |
| b) | What is Piston clearance? How it is measured? |  |
| c) | What is the use of a Ballast Resistor in an Ignition system? |  |
| d) | What is a ‘Cold Spark Plug’? |  |
| e) | What are the limits imposed upon Supercharging engines? |  |
| f) | State the reasons for high oil consumption. |  |
| Q.No.2. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Classify Engines with respect to their arrangement of valve trains. |  |
| b) | Explain with a neat sketch, the working of a CDI ignition system. |  |
| c) | Explain in brief, how ignition timing is varied. |  |
| Q.No.3. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain the Idling circuit of a Zenith Carburetor with a neat sketch. |  |
| b) | What is heating value of a fuel? State any one method to find it out. |  |
| c) | Explain in brief, how is Diesel injection timing varied. |  |
| Q.No.4. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Discuss the effects of Supercharging of S.I Engines. |  |
| b) | How are exhaust gases treated by means of the Air injection method? |  |
| c) | Mention the various methods used to control exhaust emissions by controlling A/F mixture and briefly describe each one of them. |  |
| Q.No.5. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | State the properties of an ideal lubricating oil. Explain what is sludge formation? |  |
| b) | Explain the construction and working of a Rotor lubricating oil pump. |  |
| c) | A single engine cylinder works on a 4-stroke cycle. After testing, the following data was obtained: Engine speed = 600 r.p.m Indicator area = 360mm2 Length of Indicator diagram = 55mm Spring Scale = 0.1N/mm2/mm Bore x stroke = 100mm x 125mm Calculate the I.H.P. |  |
| Q.No.6. | Answer any two of the following Questions: | 2 x 6 = 12 |
| a) | Explain with a neat sketch the construction and working of a NDIR analyzer. |  |
| b) | By the Air box method, explain how air consumption is monitored. |  |
| c) | How is F.H.P determined by using the Morse test? Explain in brief. |  |

***********